## BOARD OF TECHNICAL EDUCATION

PORVORIM-GOA

April, 2012 Examinations

Programme: Electronics & Instrumentation Engg.

Subject: Basic Instrumentation-II(S-4216)

# Time Duration: 3 Hrs.                                               Max. Marks:  75

Instructions: 1) All Questions are compulsory.

2) Figures to the right indicate full marks.

3) Assume suitable additional data if required.

| Q.No.1. | Answer the following Questions: | 5 x 3 = 15 |
| --- | --- | --- |
| a) | Compare DC and AC amplifiers. |  |
| b) | Draw the circuit of an integrator using Opamp. State one application. |  |
| c) | Explain the principle of working of a tachometer encoder. |  |
| d) | Compare electrostatic and electromagnetic deflection system in CRO. |  |
| e) | Differentiate between accuracy and precision with the help of an example. |  |
| Q.No.2. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Explain piezoelectric effect. Draw the equivalent circuit. List two materials exhibiting this effect. |  |
| b) | Describe the construction of fibre optic transducer and explain its application in measurement of level. |  |
| c) | Explain the principle and construction of incremental encoder and absolute encoder. Compare the utility of both. |  |
| Q.No.3. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Suggest a suitable signal conditioning circuit for Resistance Temperature Detector (RTD). Draw the circuit diagram and explain the procedure for measurement. |  |
| b) | Explain one methods of Digital to Analog conversion. |  |
| c) | Explain why V-to-I converters are used in instrumentation. Draw the circuit diagram of V-to-I converter using Opamps. |  |
| Q.No.4. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Draw the block diagram of a potentiometric recorder and explain the working. |  |
| b) | Explain the construction and working of a Magnetic tape recorder. |  |
| c) | State the advantages of active filter over passive filter. Draw a circuit diagram of an active low pass filter and state the expression for frequency. |  |
| Q.No.5. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Explain the following terms: i)Resolution ii)Drift iii)Linearity. |  |
| b) | Explain the grounding and shielding techniques used in instrumentation. |  |
| c) | Describe the frequency response of first order system. |  |
| Q.No.6. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Explain the construction and working of an electrical Tachometer. |  |
| b) | Explain the following terms and their significances in instrumentation. State typical specifications. i)Hysterisis ii)Sensitivity |  |
| c) | Draw the circuit diagram of a differential amplifier using Opamp. State the drawback and explain how it is overcome. |  |

***********