## BOARD OF TECHNICAL EDUCATION

PORVORIM-GOA

April, 2012 Examinations

Programme: Mechanical Engg.

Subject: Manufacturing Process-II(4036)

# Time Duration: 3 Hrs.                                               Max. Marks:  75

Instructions: 1) All Questions are compulsory.

2) Figures to the right indicate full marks.

3) Assume suitable additional data if required.

| Q.No.1. | Answer any five of the following Questions: | 5 x 3 = 15 |
| --- | --- | --- |
| a) | What is the use of a rest in turning operation? State the difference between steady rest and follower rest. |  |
| b) | Draw a neat sketch of a chip breaker and state its use. |  |
| c) | What is the difference between face milling and end milling? |  |
| d) | Draw a neat labelled sketch showing clamping of a thin work piece in a Shaper’s vice. |  |
| e) | What is the difference between buffing and polishing? |  |
| f) | State the uses of the various die accessories used in conjunction with the dies and punches. |  |
| g) | Differentiate between coining and embossing. |  |
| Q.No.2. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Explain the difference between orthogonal and oblique cutting method with sketches. |  |
| b) | What is a taper? Explain taper turning by swivelling the compound slide on a centre lathe. |  |
| c) | What is a mandrel? Sketch and explain different types of mandrels. |  |
| Q.No.3. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | With a neat sketch explain mounting of a grinding wheel. |  |
| b) | Name at least three products whose surface finishing is done by the following processes: i) Honing ii) Lapping iii) Buffing iv) Electroplating v) Galvanizing vi) Cylindrical grinding. |  |
| c) | What is a grinding wheel? Discuss various types of abrasives used in making a grinding wheel. |  |
| Q.No.4. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | With a neat sketch explain the construction and working of a fly-press. |  |
| b) | Explain in your own words, how a work piece is held inbetween centres on a centre lathe. |  |
| c) | With a neat sketch explain the working of a hydraulic shaper. |  |
| Q.No.5. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Explain the following operations on a shaper: - Machining of inclined surfaces - Machining of slots and grooves. |  |
| b) | Discuss the various methods of holding tools in the spindle of a drilling machine. |  |
| c) | Explain how the head of a hexagonal bolt is machined on a milling machine. |  |
| Q.No.6. | Write short notes on any three: | 3 x 4 = 12 |
| a) | Radial drilling machine b) Eccentric turning |  |
| c) | Function of cutting fluids d) Milling machine attachment |  |
| e) | Method of die support |  |

***********