## BOARD OF TECHNICAL EDUCATION

PORVORIM-GOA

April, 2012 Examinations

Programme: Mechanical Engg.

Subject: Power Plant Engineering(5037)

# Time Duration: 3 Hrs.                                                Max. Marks: 75

Instructions: 1) All Questions are compulsory.

2) Figures to the right indicate full marks.

3) Assume suitable additional data if required.

| Q.No.1. | Answer any five of the following Questions: | 5 x 3 = 15 |
| --- | --- | --- |
| a) | Define the following terms: i) Plant use factor ii) Load factor iii)Demand factor. |  |
| b) | How are the hydro-electric power plants classified? |  |
| c) | What are the different fields where the use of diesel power plant is essential? |  |
| d) | Write three major advantages of Gas turbine plant over Diesel plant. |  |
| e) | Why is the feed water treatment necessary before supplying to the boiler? |  |
| f) | Sketch schematic diagram of a condensing steam power plant and explain the sequence of operations. |  |
| g) | How are the radio active wastes disposed? |  |
| Q.No.2. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | A power station supplies the following loads to the consumers. Time in hours; 0-6; 6-10; 10-12; 12-16; 16-20; 20-22; 22-24. Load in MW; 20; 50; 60; 40; 80; 70; 40. i)Find the load factor of the plant. ii) What is the load factor of a stand by equipment of 20MW capacity if it takes up all loads above 60 MW? |  |
| b) | With the help of a flow sheet, explain the working of storage type hydro-electric power plant. |  |
| c) | Draw a neat sketch of medium capacity diesel power plant and explain briefly. |  |
| Q.No.3. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Explain the working of costant pressure open cycle gas turbine power plant. |  |
| b) | With the help of a neat sketch, explain the various steps involved in the preparation of coal. |  |
| c) | Discuss the various factors to be considered during site selection of a thermal power plant. |  |
| Q.No.4. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | With the help of a neat sketch explain the construction and working of an V-flame burner. |  |
| b) | Explain the Demineralization process of water treatment. |  |
| c) | Draw a general layout of ash handling plants and explain. |  |

………2/-

-2-

| Q.No.5. | Answer any two of the following Questions: | 2 x 6 = 12 |
| --- | --- | --- |
| a) | With the help of neat sketch explain the construction and working of an electrostatic precipitator. |  |
| b) | With the help of a neat sketch explain the construction and working of a hyperbolic cooling tower. State its merits and demerits. |  |
| c) | Draw a neat sketch of a breeder reactor. State its advantages and disadvantages. |  |
| Q.No.6. | Write short notes on any Three: | 3 x 4 = 12 |
| a) | Functions of feedwater loop |  |
| b) | Site selection for hydroelectric power plant |  |
| c) | I.C. engine cooling system |  |
| d) | Nuclear fuels used in reactors |  |
| e) | Prevention of carryover losses in cooling towers |  |

***********