## BOARD OF TECHNICAL EDUCATION

PORVORIM-GOA

April, 2012 Examinations

Programme: Shipbuilding Engg.

Subject: Marine Engineering-I(4336)

# Time Duration: 3 Hrs.                                                Max. Marks: 75

Instructions: 1) All Questions are compulsory.

2) Figures to the right indicate full marks.

3) Assume suitable additional data if required.

| Q.No.1. | Answer any five of the following Questions: | 5 x 3 = 15 |
| --- | --- | --- |
| a) | What is boiler water treatment and why it is necessary? |  |
| b) | What will be the pipe material and type of joints for high pressure steam supply? |  |
| c) | Make a list of the boiler mountings for a shipboard boiler. What is the difference between mountings and accessories? |  |
| d) | Briefly discuss the centrifugal pump characteristics. |  |
| e) | What are the safety devices mounted on a Air compressor? How does a intercooler improve the efficiency of the compressor? |  |
| f) | Why is priming of centrifugal pumps necessary? List a few advantages of centrifugal pumps. |  |
| Q.No.2. | Answer any one of the following Questions: | (12) |
| a) | What is a nonrising stem Gate valve? Sketch and describe and state its uses onboard a ship. |  |
| b) | What are the advantages of a Butterfly Valve? Describe with a neat sketch a Butterfly valve. |  |
| Q.No.3. | Answer any one of the following Questions: | (12) |
| a) | What is a reciprocating pump? Where is a reciprocating pump used onboard a ship? Describe the same using a neat sketch. |  |
| b) | Briefly describe i) Testing of pumps ii) Pump maintenance required. |  |
| Q.No.4. | Answer any one of the following Questions: | (12) |
| a) | Describe step wise the process of raising pressure in a boiler, after it has been shutdown for maintenance. List a few maintenance activities of a boiler. |  |
| b) | Sketch and describe an Auxilliary boiler used on motor ships. List a few maintenance activities in such a boiler. |  |
| Q.No.5. | Answer any one of the following Questions: | (12) |
| a) | How do the unloading of Air compressor valves takes place? With a simple sketch explain the same. List the preventive measure taken prior to starting and during running of the Air compressor. |  |
| b) | What are the important advantages of multistaging? With a neat sketch describe the constructional features and working of a two stage reciprocating air compressor. |  |
| Q.No.6. | Answer any one of the following Questions: | (12) |
| a) | With simple sketches briefly describe the different methods of joining pipes. What types of jointings are used on pipelines carrying diesel fuel, steam and acids? |  |
| b) | Briefly explain any 3 of the following: i) Mud box ii) Storm Valve iii) Application of pumps onboard ships iv)Deck piece. |  |

***********