## BOARD OF TECHNICAL EDUCATION

PORVORIM-GOA

April, 2012 Examinations

Programme: Automobile Engg.

Subject: Hydraulic Machinery(4053)

# Time Duration: 3 Hrs.                                                Max. Marks: 75

Instructions: 1) All Questions are compulsory.

2) Figures to the right indicate full marks.

3) Assume suitable additional data if required.

| Q.No.1. | Answer any five of the following Questions: | 5 x 3 = 15 |
| --- | --- | --- |
| a) | Define the term: i)Specific gravity ii)Specific volume. |  |
| b) | Explain with mathematical expression the relationship between vacuum pressure and absolute pressure. |  |
| c) | State two important characteristics of turbulent flow. |  |
| d) | What do you understand by the term ‘Centre of pressure’. |  |
| e) | What is priming? State various methods priming. |  |
| f) | Differentiate between a gear pump and a vane pump. |  |
| g) | The intensity of pressure of water at a point is 400 KPa. Express the pressure in terms of head of water in metres. |  |
| Q.No.2. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | With a neat sketch explain the working of micro manometer. |  |
| b) | Explain the term ‘Slip’ and ‘Cavitation’. State the functions of air vessel in a reciprocating pump. |  |
| c) | Describe with the help of a neat sketch the working of a hydraulic accumulator. |  |
| Q.No.3. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Define static and manometric head of a centrifugal pump. State & explain the different types of head losses which may occur in a pump installation. |  |
| b) | A circular plate 2.5 m diameter is immersed in water, its greatest and least depth below the free surface being 3m and 1m respectively. Find - Total pressure on one face of the plate - Position of centre of pressure. |  |
| c) | Describe with the help of a neat sketch the construction and working of a Plunger pump. |  |
| Q.No.4. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Find the discharge through a right angled triangular notch, under a head of 0.25 m. Assume Cd=0.6. |  |
| b) | Find the loss of head due to friction in a pipe 1500m long and 2.5cm in diameter. The velocity of water in pipe is 0.70 m/s. Take the Darcy coefficient as 0.004. |  |
| c) | A Venturimeter is to be fitted in a pipe 0.25m diameter where the pressure head is 7.6m of flowing liquid and the flow is 0.135 m3/s. Find the diameter of the throat, so that the pressure head does not become negative. Take K =0.96. |  |

………2/-

-2-

| Q.No.5. | Answer any two of the following Questions: | 2 x 6 = 12 |
| --- | --- | --- |
| a) | With a neat sketch explain the construction and working of Hydraulic Press. |  |
| b) | Differentiate between a centrifugal pump and a reciprocating pump considering at least four aspects. |  |
| c) | A jet of water issued from a sharp edged vertical orifice under a constant head of 0.51m. At a certain point of issuing jet, the horizontal and vertical co-ordinates measured from the vena-contracta are 0.406m and 0.085m respectively. Determine Cv. |  |
| Q.No.6. | Write short notes on any three: | 3 x 4 = 12 |
| a) | Water hammer |  |
| b) | Submersible pump |  |
| c) | Piezometer |  |
| d) | Bernoulli’s theorem & equation |  |

***********