## BOARD OF TECHNICAL EDUCATION

PORVORIM-GOA

April, 2012 Examinations

Programme: Electronics Engg./EC/MED/E&EE

Subject: Circuits & Networks(4136)

# Time Duration: 3 Hrs.                                                Max. Marks: 75

Instructions: 1) All Questions are compulsory.

2) Figures to the right indicate full marks.

3) Assume suitable additional data if required.

| Q.No.1. | Answer any five of the following Questions: | 5 x 3 = 15 |
| --- | --- | --- |
| a) | Define the terms: Active and passive elements. State examples of each. The equivalent of 2 series connected resistances is 50 and when connected in parallel, it is 12. Find the individual resistance values. |  |
| b) | With the help of neat diagram, explain Kirchoff’s laws, along with statements. |  |
| c) | If V=20+15j, I=4-3j, find impedance, resistance and reactance in the circuit. |  |
| d) | Explain the operation of RC integrator circuit with expression. What other properties does it exhibit? |  |
| e) | Define characteristics impedance. Assume any symmetrical network & calculate its characteristics impedance. Verify that product of open and short circuit impedances equals square of its characteristic impedance. |  |
| f) | “Attenuation in db is equal to 8.68 times attenuation is Neper”. Justify this statement. |  |
| Q.No.2. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Using nodal analysis, find the voltage V which makes current in 10 resistance zero, in figure No.1. |  |
| b) | Calculate current in 4 resistor using mesh analysis, in figure No.2. |  |
| c) | Explain the terms: i)Ideal voltage source ii) Ideal current source. Determine equivalent capacitance as viewed from voltage source as in figure No.3. |  |
| Q.No.3. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Calculate current in 4 resistor using Thevenin’s theorem, in figure No.4. |  |
| b) | Calculate current in 10 resistor using superposition method in figure No.5. |  |
| c) | Using Norton method, calculate current in 1 resistor in figure No. 6. |  |
| Q.No.4. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Refer figure No.7. If V=10 30, find V0. |  |
| b) | Refer figure No.7. If V0=5 0, find V. |  |
| c) | Explain the behaviour of a RC circuit, to which ac voltage is applied. 50V a.c. at line frequency is applied to series RC Circuit, with R=100, C=30F. Find i) Total impedance ii) Current iii)Power factor iv) True power v) Reactive power v) apparent power. |  |
| Q.No.5. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Derive the expression for bandwidth in series RLC circuit, at resonance. Also obtain the relation between Q factor, bandwidth and resonant frequency. |  |

………2/-

-2-

| b) | A series RC circuit with R=2.7K, C=2.2F has a 10V dc applied to it at t=0. Obtain current, voltage across R and C at t=0.1s, t=1s and t=5s. Sketch these waveforms. |  |
| --- | --- | --- |
| c) | A series RL circuit with R=1K, L=250mH has a 10V dc applied to it at t=0. Obtain the values of current, voltage across R and L at t=0.1s, t=1s and t=5s. Sketch these waveforms. |  |
| Q.No.6. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Convert i) 3 nepers into decibels ii) 2 decibels into nepers. Following measurements were made on a symmetrical network. Zoc=200, Zsc=150. Obtain the equivalent T network. Convert this into equivalent  and verify that terminal impedances measured is the same. |  |
| b) | Design symmetrical attenuator to provide 25db attenuation when working with 600 impedances. Design prototype filter with design impedance of 600 which will allow all frequencies from 10KHz to 14KHz to pass. |  |
| c) | Write notes on i)Classification of filters ii)Equalizers. |  |