## BOARD OF TECHNICAL EDUCATION

PORVORIM-GOA

April, 2012 Examinations

Programme: Automobile Engg.

Subject: Automobile Engines-I(4054)

# Time Duration: 3 Hrs.                                               Max. Marks:  75

Instructions: 1) All Questions are compulsory.

2) Figures to the right indicate full marks.

3) Assume suitable additional data if required.

| Q.No.1. | Answer any five of the following Questions: | 5 x 3 = 15 |
| --- | --- | --- |
| a) | Sketch ideal P.V and T-S diagrams for the Otto cycle and explain them. |  |
| b) | List the additives added to petrol and briefly explain their uses. |  |
| c) | Compare the mechanical and electrical gasoline feed pumps w.r.t their merits and limitations. |  |
| d) | What is the function of a thermostat? Sketch a Dole type thermostat. |  |
| e) | Give a broad classification of I.C Engines. |  |
| f) | During idling of an S.I engine a rich air-fuel mixture is required. Why? |  |
| g) | What are cam-ground pistons? |  |
| h) | What is the function of a radiator and its cap? List various types of cores for radiator. |  |
| Q.No.2. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | What is scavenging? With a neat labelled diagram explain the working of Cranckcase scavenged engine. |  |
| b) | Drawing the valve timing diagram for 4-stroke SI engine, describe the same. Why does suction process start before TDC? |  |
| c) | Explain the construction and functions of a cylinder block. What is monoblock? |  |
| Q.No.3. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Describe with a neat sketch the refining of petroleum by Fractional distillation. |  |
| b) | Explain with sketches the operation of Petrol Injection System in SI engines. |  |
| c) | What air-fuel mixture ratio is required for acceleration in a carburettor and why? Using a neat sketch explain the acceleration circuit in a carburettor. |  |
| Q.No.4. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | What is the necessity of cooling an engine? Describe the air cooling method and its advantages and disadvantages. |  |
| b) | Explain with sketches the Diesel Injection Nozzles and their specific uses. |  |
| c) | Describe the common rail and the distributor type Fuel Injection systems. |  |

……2/-

-2-

| Q.No.5. | Answer any two of the following Questions: | 2 x 6 = 12 |
| --- | --- | --- |
| a) | What are the objectives of I.C engine lubrication? List the requirements of a good engine lubricant and describe them. |  |
| b) | Sketch various CI Engine combustion chambers and explain any two of them in detail. |  |
| c) | Explain ‘detonation’ in SI engines and its effects. |  |
| Q.No.6. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Explain the Mist Lubrication system and its advantages and disadvantages. |  |
| b) | Draw the performance curves for a petrol engine and comment on them. |  |
| c) | i)What do you mean by Indicated Power? How is its formula derived? ii)A 2- Cylinder, 4-stroke petrol engine was tested on a Rope Brake dynamometer having drum diameter as 40cm and Rope diameter as 2.5 cm. The difference in tensions on light and slack sides of the rope was 3.5 kg. If the speed was 60 rps(rev/sec), calculate the B.P of the engine. |  |

***********