## BOARD OF TECHNICAL EDUCATION

PORVORIM-GOA

April, 2012 Examinations

Programme: Electrical Engg./E&EE

Subject: Utilisation of Electrical Energy(4111)

# Time Duration: 3 Hrs.                                               Max. Marks:  75

Instructions: 1) All Questions are compulsory.

2) Figures to the right indicate full marks.

3) Assume suitable additional data if required.

| Q.No.1. | Answer any five of the following Questions: | 5 x 3 = 15 |
| --- | --- | --- |
| a) | List any six methods of Electric heating. |  |
| b) | List the properties a good heating element should posses. |  |
| c) | Which type of DC generator is preferred for ARC welding? Why? |  |
| d) | With neat diagram explain spot welding. |  |
| e) | With neat diagram explain Flash welding. |  |
| f) | For electric traction define i)Crest speed ii)Scheduled speed. |  |
| g) | Give classification of luminaire. |  |
| f) | Define (i) Luminous intensity (ii) Luminance. |  |
| Q.No.2. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | What is indirect resistance heating? Explain construction and working of any one furnace using indirect resistance heating. |  |
| b) | Explain infrared heating. State its advantages and applications. |  |
| c) | i)Explain any three causes of failure of heating element. ii)Explain effect of supply voltage variation on heat generated by heating element. |  |
| Q.No.3. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Explain principle of Arc welding? State & briefly explain requirements of arc welding source. |  |
| b) | State the methods of current control in an AC are welding set. Explain any two such methods. |  |
| c) | i)Explain importance of straight and reverse polarity in arc welding. ii)Explain the need of drooping characteristics for welding generator. How it is achieved in AC welding set. |  |
| Q.No.4. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | With neat diagram explain - The bow collector - Pentograph collector. |  |
| b) | With neat diagram explain bridge transition method. |  |
| c) | Explain different speed control methods employed for traction motors. |  |
| Q.No.5. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | With a neat diagram explain any one core type induction furnace. State in limitations. |  |
| b) | Explain construction and working of any one Arc furnace. State in application. |  |
| c) | Estimate the heat developed by a 20 KW, 230 Volt, single phase resistance oven. The wire temperature should not exceed 1200C and temperature of charge is to be 600C. Assume radiating efficiency =0.6 & Emmissivity = 0.9. | ….2/- |

-2-

| Q.No.6. | Answer any two of the following Questions: | 2 x 6 = 12 |
| --- | --- | --- |
| a) | With a neat diagram explain construction and working of low pressure sodium vapour lamp. Can it be used in residences or offices? |  |
| b) | Explain importance of following in lighting design - Absorption factor - Maintenance factor. |  |
| c) | Write a short note on - Colour rendering - Stroboscopic effect. |  |

***********