## BOARD OF TECHNICAL EDUCATION

PORVORIM-GOA

April, 2012 Examinations

Programme: Civil Engg./CT

Subject: Analysis of Structures(4026)

# Time Duration: 3 Hrs.                                               Max. Marks:  75

Instructions: 1) All Questions are compulsory.

2) Figures to the right indicate full marks.

3) Assume suitable additional data if required.

| Q.No.1. | Answer any five of the following Questions: | 5 x 3 = 15 |
| --- | --- | --- |
| a) | Define Middle third Rule and Middle Quarter rule. |  |
| b) | Define Principal planes and Principal stresses. |  |
| c) | Derive expression for maximum deflection for a cantilever beam carrying a point load at free end. Use Moment-Area Theorem method. |  |
| d) | Define Distribution Factor and Distribution Theorem in moment distribution analysis. |  |
| e) | State the advantages and disadvantages of a Fixed beam. |  |
| f) | State the limitations of Euler’s Formula. |  |
| g) | Define Equivalent Transformed section with examples. |  |
| Q.No.2. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | A flitched beam consists of timber joist 140mm wide x 300mm deep with a steel plate of 140mm wide and 12mm thick connected to top and bottom of wooden beam. If stress in steel is 120N/mm2, find stress in wood and also moment of resistance of beam. |  |
| b) | A Bridge pier of 1500mm wide, 1800mm deep carry an eccentric load of 1200KN on its diagonal in 1st quadrant at a distance of 400mm from axis of column. Determine stress produced at all corners of column. |  |
| c) | A concrete block 2m x 2m weighs 150KN. It carries a load of 80KN at a point on xx-axis but eccentric about y-y axis. Find the eccentricity if stresses on the edge nearer to load is twice the stress on other side of edge. Also find these stresses. |  |
| Q.No.3. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | At a point in a strained material direct normal stresses are 120N/mm2 (tensile) and 50N/mm2 (compressive) acts along with shear stress of 35N/mm2 across both planes. Determine resultant stress on a plane of 30 with major stress. |  |
| b) | A line shaft is subjected to bending tensile stress of 110N/mm2 along longitudinal axis along with transverse shear stress of 60N/mm2. Determine the maximum and minimum principal stresses along with their planes. |  |
| c) | When a thin-walled tube is subjected to internal pressure and torque. The stresses in tube wall are 60N/mm2 tensile, 30N/mm2 tensile in a direction at right angles to first stress and also a shear stress of 15N/mm2 across both planes. Determine principal stress and maximum shear stress in magnitude and direction. Use Mohr’s Circle method. |  |

………2/-

-2-

| Q.No.4. | Answer any one of the following Questions: | 1 x 12 = 12 |
| --- | --- | --- |
| a) | A simply supported beam of 250mm wide and 400mm deep carry point loads of 20KN, 30 KN and 10 KN at distances of 2m, 4m and 6m from left end of beam of span 8m. Determine slope at both supports and deflection under all load points. Take E= 0.18 x 105 N/mm2. |  |
| b) | A fixed beam of uniform section carry two concentrated loads of 40KN and 25KN at distances of 2m and 4m from left support and also a UDL of 15KN/m between point loads. Determine Fixed End moments and draw SFD and BMD of beam. EI is uniform throughout. |  |
| Q.No.5. | Answer any one of the following Questions: | 1 x 12 = 12 |
| a) | A continuous beam ABC has span AB=6m and BC=4m with supports A fixed and B and C as simple support. A UDL of 30KN/m on AB span and a point load of 20KN at 1m from end C on BC span. Flexural rigidity of AB=2EI while BC=1.5 EI. Analyse beam for support moments by Moment Distribution method and draw SFD and BMD of beam. |  |
| b) | Analyse continuous beam ABCD with all supports as simple support. AB=6m, BC=4m and CD=3m. UDL of 30KN/m acts on AB span, a point load of 35 KN on BC span at centre and UDL of 20KN/m on CD span. EI is uniform throughout. Analyse beam by moment Distribution method and sketch SFD and BMD. |  |
| Q.No.6. | Answer the following Questions: | 2 x 6 = 12 |
| a) | A round steel rod of 25mm and 1.5m length is subjected to axial compressive load. Using Enlev’s formula find Buckling load. Also find maximum lateral deflection corresponding to buckling condition. Both ends of rod are hinged. Take E=2x105 N/mm2 and yield stress of steel = 250 N/mm2. |  |
| b) | A steel column of I section of flanges 150 x 12mm and web 10 x 276mm is 4m long with one end fixed and other hinged. Determine safe load of column. Use Rankine’s formula. Take crushing stress=330 N/mm2 and Rankine’s constant = 1/7500 and Factor of safety as 2.5. |  |

***********