## BOARD OF TECHNICAL EDUCATION

PORVORIM-GOA

April, 2012 Examinations

Programme: Fabrication Technology & Erection Engg.

Subject: Fabrication Estimation & Costing(4242)

# Time Duration: 3 Hrs.                                                Max. Marks: 75

Instructions: 1) All Questions are compulsory.

2) Figures to the right indicate full marks.

3) Assume suitable additional data if required.

4) Use of Structural Engineers Hand book SP:6(1) or

steel table is permissible.

| Q.No.1. | Answer any five of the following Questions: | 5 x 3 = 15 |
| --- | --- | --- |
| a) | Explain in short the procedure to determine the labour cost for welding operation. |  |
| b) | Define a detailed estimate and an approximate estimate. |  |
| c) | Define cutting speed, feed and depth of cut in machinery operations. |  |
| d) | Differentiate between Beyond economic repairs and obscolescence in equipment replacement. |  |
| e) | Calculate the material cost to manufacture a ring of structural steel of outer diameter 400 mm and rod diameter of 15mm. |  |
| f) | Define (i) Breach of contract (ii) Arbitration. |  |
| g) | Differentiate between Security Deposit and Earnest Money Deposit. |  |
| Q.No.2. | Estimate for the quantities of 1. Structural steel 2. A.C. Roof sheet coverings for a roof truss shown in Fig.1. The truss used are King Post Truss for a vehicle parking shed of span 5 m and length of shed is 45 m, with spacing of truss 3 m. Neglect gusset plates. NOTE for Fig.1:- i) All members of truss are ISA 80 x 80 x 6.0 mm @ 7.1 Kg/m ii) AB = BC. | (12) |
| Q.No.3. | Answer the following Questions: |  |
| a) | Calculate the time required to machine the job shown in Fig.2 of Brass machined from a 60mm diameter round stock. Also calculate the labour cost if the machine operators are paid at Rs.50/- per hour. (assume suitable machining parameters). | (08) …..2/- |

-2-

| b) | If the machines employed to machine the above job, shown in Fig.2 (Q2a), has a motor of 2HP, calculate the electric power charge utilized to machine the job. The electric power charge rate is Rs. 10/unit. | (04) |
| --- | --- | --- |
|  | ### OR |  |
| b) | Explain one method to estimate the sheet metal operation timing. | (04) |
| Q.No.4. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | With a neat sketch, explain the Break Even theory. Also, explain how this theory could be used for carrying out equipment replacement studies. |  |
| b) | Calculate the power cost for welding, with a diesel engine driven generator per hour, if the are voltage employed were 25 Volts, arc current of 120 Amps, fuel consumption of 5ml of diesel per HP of power developed and cost of diesel is Rs. 4//litre. |  |
| c) | A Hydraulic press was purchased by a company for Rs. 12,50,000/-,with an expected useful life of 6,00,000 strokes. The press works at the rate of 3 strokes/minute and consumes 5 units/hr of electrical energy at the rate of Rs.10/unit. Calculate the cost of a single stroke of the press. |  |
| Q.No.5. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | List the different components of cost of a product and explain each of them shortly. |  |
| b) | A machine was purchased by a company for Rs.5,20,000/- with a expected useful life of 10 years and salvage value of Rs.52,000/-. Calculate the depreciated amount at the end of sixth year, by Reducing Balance method. |  |
| c) | What is Estimation? Explain the procedure to be followed to perform estimation of a product. Differentiate between Estimation and Costing of a product. |  |
| Q.No.6. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Briefly explain different types of contracts with advantages and disadvantages. |  |
| b) | Explain the following conditions of contract: i)Time limits ii)Terms of payments iii)Tender notice iv)Execution of work. |  |
| c) | Prepare a measurement page for the roof truss as shown in Fig.3, for the items of structural steel members. NOTE FOR FIG.3: Inside members of Truss are ISA 50 x 50 x 6.0mm @ 4.4 Kg/m. |  |

……….3/-