## BOARD OF TECHNICAL EDUCATION

PORVORIM-GOA

April, 2012 Examinations

Programme: Mechanical Engg./P

Subject: Tool Engineering-II(Press Tools)[5077]

# Time Duration: 3 Hrs.                                               Max. Marks:  75

Instructions: 1) All Questions are compulsory.

2) Figures to the right indicate full marks.

3) Assume suitable additional data if required.

| Q.No.1. | Design and draw (at least two views) a die to produce a sheet metal part 3 mm thick made of Al alloy with ultimate shear strength 220 N/mm2. Refer fig.1. | (40) |
| --- | --- | --- |
|  | The design should include: |  |
|  | - Press tonnage calculation |  |
|  | - Economic strip layout |  |
|  | - Bill of material. |  |
| Q.No.2. | Answer any two of the following Questions: | 2 x 10 = 20 |
| a) | Explain pilots, strippers and pressure pads with sketches. |  |
| b) | What is a progressive die? What are its advantages? Sketch a work piece for which progressive die can be used. |  |
| c)i) | With a neat sketch explain the complete bending terminology. How is bending allowance calculated? |  |
| ii) | Explain springback. |  |
| d)i) | Explain defects in deep drawn parts. |  |
| ii) | Calculate the blank diameter to draw a symmetrical cup with shell height 50 mm  and shell diameter 50 mm. Assume material thickness 3 mm. |  |
| Q.No.3. | Write short notes on any three: | 3 x 5 = 15 |
| a) | Embossing and coining |  |
| b) | Stretch forming |  |
| c) | Rubber dies |  |
| d) | Economic strip layout |  |
| e) | Variables that affect metal flow during drawing |  |
| f) | Vee-bending dies |  |