## BOARD OF TECHNICAL EDUCATION

PORVORIM-GOA

April, 2012 Examinations

Programme: Electronics & Instrumentation Engg.

Subject: Basic Control Engineering(4219)

# Time Duration: 3 Hrs.                                                Max. Marks: 75

Instructions: 1) All Questions are compulsory.

2) Figures to the right indicate full marks.

3) Assume suitable additional data if required.

| Q.No.1. | Answer any five of the following Questions: | 5 x 3 = 15 |
| --- | --- | --- |
| a) | Briefly explain multivariable control system. |  |
| b) | Describe shaft encode and list three commonly used types. |  |
| c) | What is meant by stable and unstable rotor teeth position in permanent magnet stepper motor? |  |
| d) | Draw the schematic of D.C. motor and also show representation of its Transfer function in the form of block diagram. |  |
| e) | Given a second order servomechanism with K = 9, J =1 and F=9. Find the natural frequency and damping ratio. |  |
| f) | Define: i) Marginally stable ii) Conditionally stable. |  |
| Q.No.2. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Explain in detail, various effects of using feedback in control systems. |  |
| b) | Give an example of position control system and with the help of a diagram, explain the control technique. |  |
| c) | Explain the concept of voltage build up in a Shunt Generator. |  |
| Q.No.3. | Answer any three of the following: | 3 x 4 = 12 |
| a) | With a diagram, explain A.C. tachometer. |  |
| b) | Explain linear and rotary POT using a schematic diagram. |  |
| c) | Discuss the effect of addition of poles and zeros on the root locus. |  |
| d) | Find the Laplace Inverse of X(s)=s+5s2(s+1). |  |
| Q.No.4. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | For a system with block diagram shown in fig 4.1, find (i) DTG (ii) OLTF (iii) CLTF and draw reduced block diagram (iv) Characteristic equation (v) Actuating signal ratio. |  |
| b) | In the block diagram of fig 4.2, (i) Find DTG (ii) What is the value at point A in terms of C? (iii) Find loop gain (iv) Find C/R. |  |
| c) | A unity feedback system has G(s)=1s(1+5s)(1+2s). Determine the steady state errors for unit ramp, unit step and unit acceleration input. |  |

………2/-

-2-

| Q.No.5. | Answer the following Questions: | 2 x 6 = 12 |
| --- | --- | --- |
| a) | Explain the schematic of a Second Order Servo System. |  |
| b) | Draw the block diagram of the system components associated with the schematic of 2nd Order Servo System. |  |
| Q.No.6. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | State the Routh’s Stability Criterion. A 4th order system is represented by the characteristic equation s4+8s3+18s2+16s+15=0. Using Routh Criterion, determine the stability of the system. |  |
| b) | Draw the Nyquist Plot for G(s)H(s)=s+2s(s+1)(s+2) |  |
| c) | For a Control System, the OLTF is G(s)=2s(s+1)(s2+1). Draw the bode plot and determine Gain and phase margin. |  |