## BOARD OF TECHNICAL EDUCATION

PORVORIM-GOA

April, 2012 Examinations

Programme: Mechanical Engg./P/A/SB/TD

Subject: Strength of Materials(4032)

# Time Duration: 3 Hrs.                                                Max. Marks: 75

Instructions: 1) All Questions are compulsory.

2) Figures to the right indicate full marks.

3) Assume suitable additional data if required.

| Q.No.1. | Answer any five of the following Questions: | 5 x 3 = 15 |
| --- | --- | --- |
| a) | Derive the equation for the strain energy stored in a body due to gradually applied load. |  |
| b) | A steel wire of 6mm diameter is bent into a circular shape of 6m radius. Determine the maximum stress induced in the wire. E=200 GPa. |  |
| c) | Explain the various types of stresses in cylindrical shells. |  |
| d) | A steel rod 6m long and 45mm in diameter is used as a column with one end fixed and other end free. Determine the crippling load by Eulers formula. E=200 GPa. |  |
| e) | What do you understand by Eccentric loading? Derive the equation for maximum stress induced in a body subjected to direct stress and bending stress. |  |
| f) | State and prove the parallel axis theorem. |  |
| g) | A stainless steel beam of 4m is subjected to a central load of 12KN. Find the maximum slope & deflection of the beam. I=12x106mm4, E=200 GPa. |  |
| Q.No.2. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Explain the stress-strain diagram with a neat sketch |  |
| b) | An aluminium tube of 50mm outer diameter and 30mm inner diameter is tightly fitted on to a solid steel rod of 30mm diameter. The composite bar is loaded in compression by an axial load ‘P’. Find the stress in aluminium when the stress in steel is 70N/mm2. Esteel= 200KN/mm2 and Ealuminium=70KN/mm2. Also find total load. |  |
| c) | A rod of steel is 25m long & at a temp of 30C. Find the free expansion of the rod when the temperature is raised to 75C. Also, find the temperature stresses produced when - Expansion of the rod is prevented - When the rod is permitted to expand by 6.2mm = 12x10-6/C and E=2x105 N/mm2. |  |
| Q.No.3. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | A point is subjected to a tensile stress of 55N/mm2 and a compressive stress of 45N/mm2 acting on 2 mutually perpendicular planes and a shear stress of 12N/mm2 on the plane. Determine the principal stresses and maximum shear stress. |  |
| b) | A 2.5m long bar of 1600mm2 crossectional area hangs vertically and has a collar fixed at its lower end. Find the stress induced in the bar when a weight of 3KN falls from a height of 150mm on the collar. Also find the strain energy stored in the bar. E=120GPa. |  |
| c) | A cantilever beam 2m long carries point loads of 4KN,5KN and 6KN at distances of 0.5m, 1m and 1.5m respectively from the free end. Calculate the Shear Force and Bending Moment and also draw them. | ……2/- |

-2-

| Q.No.4. | Answer any two of the following Questions: | 2 x 6 = 12 |
| --- | --- | --- |
| a) | A simply supported beam 8m long has an uniformly distributed load of 5KN/m over a length of 2m from the left support and an uniformly distributed load of 2KN/m over length of 3.5m from the right support. The beam has a point load of 6KN acting at the mid span. Find the Shear Force and Bending Moment and also draw them. |  |
| b) | Determine the Moment of Inertia of an I section about its horizontal centroidal axis given the dimensions of the top flange = 250x30mm, bottom flange = 150x25mm and web = 300x30mm. |  |
| c) | Find the safe concentrated load that can be applied at the free end of 3m cantilever beam. The section of the cantilever beam is hollow with outer diameter = 70mm & inner diameter = 50mm. The safe bending stress for the material is 70 N/mm2. |  |
| Q.No.5. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | A tensile stress specimen diameter 15mm is in the line of pull parallel to the axis of the specimen but is displaced from it. Determine the distance of pull from the axis when the maximum stress is 15% greater than the mean stress on a section normal to the axis. |  |
| b) | A 80mm diameter shaft is subjected to a torque of 5KN-m. Find i) Maximum shear stress in the shaft ii)Angle of twist per metre length of the shaft iii) Shear stress at a distance of 9mm from the axis of the shaft G = 8x104N/mm2. |  |
| c) | Determine the diameter of a solid shaft, which will transmit 95KW at 9500 rps if the shear stress in the shaft is limited to 65N/mm2. Also find the length of the shaft if the twist must not exceed 1 over the entire length of the shaft. G=8x104N/mm2. |  |
| Q.No.6. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | A thin cylindrical shell of thickness 6mm and diameter 400mm is subjected to an internal pressure which produces a strain of 1/2500 in the diameter. Find this internal pressure. Also find the consequent hoop and longitudinal stress. E= 2x105N/mm2 and  =0.3. |  |
| b) | Find the greatest length of a Mild steel section 30x30mm, which can be used as a compression member with one end fixed and the other end free to carry a vertical load of 40KN. Allow factor of safety = 5,  =1/7600 and compressive stress= c=330N/mm2. |  |
| c) | Steel joint is simply supported over a span of 4.5m and carries a point load of 60KN at 1.5m from the left end. Find the position and magnitude of maximum deflection. EI = 14x1012Nmm2. |  |

***********