## BOARD OF TECHNICAL EDUCATION

PORVORIM-GOA

April, 2012 Examinations

Programme: Electrical Engg.

Subject: Testing & Maintenance of Electrical Equipments(5104)

# Time Duration: 3 Hrs.                                               Max. Marks:  75

Instructions: 1) All Questions are compulsory.

2) Figures to the right indicate full marks.

3) Assume suitable additional data if required.

| Q.No.1. | Answer any five of the following Questions: | 5 x 3 = 15 |
| --- | --- | --- |
| a) | What is the significance of the polarity test performed on a transformer? |  |
| b) | State the advantages and disadvantages of testing dc machine by brake test? |  |
| c) | State the important points to be considered regarding, care of electrical equipment during period of inactivity. |  |
| d) | Explain phasing out test for a 3 phase transformer. |  |
| e) | What are the various losses that takes place in three phase Induction motor. |  |
| f) | What is meant by Type Test, Routine Test and Acceptance Test? |  |
| g) | State the routine tests required for a synchronous generator. |  |
| Q.No.2. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | In a Hopkinson’s test on a pair of 500V, 100 KW shunt generators the following data was obtained: Auxillary supply 30A at 500V, Generator output current 200A Field currents 3.5A and 1.8A Armature circuit resistances, 0.075 each machine voltage drop at brushes, 2V (each machine). Calculate the efficiency of the machine acting as a generator. |  |
| b) | Briefly explain with a neat diagram the brake test performed on a dc machine. |  |
| c) | In a brake test, the effective load on the brake pulley was 38.1 Kg, the effective diameter of the pulley 63.5 cm and speed 12 rps. The motor took 49 A at 220 V. Calculate the horse power and the efficiency at this load. |  |
| Q.No.3. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Explain back to back test on 120KVA, 3 phase 11KV/400V delta star connected transformer. Indicate the measuring instrument ranges. |  |
| b) | Explain the measuring of insulation resistance. How is it measured for power transformer? |  |
| c) | Draw a circuit diagram with measuring instrument and their ranges for performing OC test on a 100 KVA, 11 KV/400V, Delta-star transformer. Also explain the procedure for the test. |  |
| Q.No.4. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | State advantages and disadvantages of Phantom loading. |  |
| b) | Explain with a neat diagram open circuit voltage, ratio test on a slip ring Induction motor. What is the significance of this test? |  |
| c) | Give the procedure of checks to be followed, if Induction motor fails to start. | ……2/- |

-2-

| Q.No.5. | Answer any two of the following Questions: | 2 x 6 = 12 |
| --- | --- | --- |
| a) | What are the various steps involved in the installation and commissioning of 3 phase Induction motor? |  |
| b) | State and explain the different type of losses occurring in synchronous motor. Explain how any one of these losses is determined experimentally. |  |
| c) | State the various types of direct and indirect methods used for testing of Induction motor to determine losses and efficiency. Also state advantages and disadvantages of each method used. |  |
| Q.No.6. | Write short notes on any two: | 2 x 6 = 12 |
| a) | Causes of failure, its frequency of occurrence and trouble shooting in transformers |  |
| b) | Maintenance schedule for Induction machine as per IS |  |
| c) | High voltage and Impulse voltage withstand test on transformer |  |
| d) | Revarnishing process of winding of electrical equipment |  |

***********