## BOARD OF TECHNICAL EDUCATION

PORVORIM-GOA

April, 2012 Examinations

Programme: Shipbuilding Engg.

Subject: Welding in Ship Construction-I(4332)

# Time Duration: 3 Hrs.                                                Max. Marks: 75

Instructions: 1) All Questions are compulsory.

2) Figures to the right indicate full marks.

3) Assume suitable additional data if required.

| Q.No.1. | Attempt any five of the following: | 5 x 3 = 15 |
| --- | --- | --- |
| a) | What is Lorentz force? Briefly discuss the same. |  |
| b) | What are the primary equipments required for flux shielded metal are welding? |  |
| c) | Discuss the principle of operation in brazing. |  |
| d) | With examples discuss the consumable and non-consumable electrodes. |  |
| e) | Explain the metallurgical residual stresses briefly. |  |
| f) | What is kerf in thermal cutting of metals? What are the factors that affect kerf? |  |
| Q.No.2. | Attempt any two of the following: | 2 x 6 = 12 |
| a) | Can welding replace riveting? Compare welding with riveting. |  |
| b) | Describe the stages of free flight metal transfer. |  |
| c) | What is the importance of Arc stability? Discuss any one type of Arc blow with a neat sketch. |  |
| Q.No.3. | Attempt any two of the following: | 2 x 6 = 12 |
| a) | What conclusions can be drawn from the temperature distribution in welds? With simple sketches explain the temperature distribution around a metallic arc butt weld. |  |
| b) | Describe with sketches if needed the Heat affected Zone. |  |
| c) | List the various selection factors for power sources of welding. |  |
| Q.No.4. | Attempt any two of the following: | 2 x 6 = 12 |
| a) | With simple sketch explain the Manifold system of Gas welding. |  |
| b) | With a neat sketch explain the constructional features and operation of a welding Torch. |  |
| c) | Discuss the different methods of controlling residual welding stresses. |  |
| Q.No.5. | Attempt any two of the following: | 2 x 6 = 12 |
| a) | Explain what do you understand by: (i) Spatter (ii) Slag inclusion (iii) Porosity (iv) Distortion. |  |
| b) | List the common causes for Weld metal cracking and suggest remedial action to be taken to minimize or avoid the same. |  |
| c) | Discuss the electrode coating ingredients and their functions. |  |
| Q.No.6. | Attempt any two of the following: | 2 x 6 = 12 |
| a) | What is the effect on the weld bead when no shielding gas is provided? Discuss any two types of commonly employed shielding gases. |  |
| b) | What is the principle of operation in oxy-acetylene flame cutting process? Discuss the advantages and limitations of plasma arc cutting. |  |
| c) | Write short notes on: i) Applications of Gas welding ii) Oxygen gas cylinder |  |

***********