## BOARD OF TECHNICAL EDUCATION

PORVORIM-GOA

April, 2012 Examinations

Programme: Electronics Engg./EC/MED

Subject: Circuit Design(4139)

# Time Duration: 3 Hrs.                                                Max. Marks: 75

Instructions: 1) All Questions are compulsory.

2) Figures to the right indicate full marks.

3) Assume suitable additional data if required.

| Q.No.1. | Answer any five of the following Questions: | 5 x 3 = 15 |
| --- | --- | --- |
| a) | What do you understand by resistance? Briefly explain any one type of resistor. |  |
| b) | Define inductive reactance and capacitive reactance. |  |
| c) | Explain the procedure for design of feedback circuit in RC phase shift oscillator. |  |
| d) | Compare between L and C filter. |  |
| e) | Comment on the need for biasing in brief. |  |
| f) | State some important features of a RC coupled multistage amplifier. |  |
| g) | Describe the design procedure for a RC differentiator. |  |
| h) | Draw the circuit diagram of astable multivibrator using IC5555. What do you mean by duty cycle? |  |
| Q.No.2. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | A 8 microfarad capacitor is being charged by a 400 V supply through a 0.1 M resistor. How long will it take for the capacitor to develop a potential difference of 300 V? |  |
| b) | Field windings of dc electromagnet is wound with 960 turns and has a resistance of 50. When the exciting voltage is 230V, the magnetic flux linking the coil is 0.005 Weber. Calculate self-inductance of the coil and energy stored in the magnetic field. |  |
| c) | Write a short note on emf equation of a transformer. |  |
| Q.No.3. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Explain the procedure involved in the design of voltage regulator using Zener diode. |  |
| b) | Design IC 723 voltage regulator to provide an output voltage of 5 volts and 15 volts. |  |
| c) | Design a full wave center tapped rectifier with a capacitor filter to provide a dc output of 12V with ripple less than 100 mv at load current of 500 mA. |  |
| Q.No.4. | Answer any one of the following Questions: | (12) |
| a) | With the help of a neat circuit diagram, explain the detailed design procedure of a series transistorized voltage regulator to provide a certain value of dc output voltage at a specified output current. |  |
| b) | Design a half wave controlled rectifier to give a dc output of 60V to a resistive load of 15 from 230V, 50Hz supply. Use UJT 2N2646 for triggering. |  |

………2/-

-2-

| Q.No.5. | Answer the following Questions: |  |
| --- | --- | --- |
| a) | Describe the design procedure of a single stage typical common emitter amplifier circuit using transistor to obtain a desired value of voltage gain Av. | (8) |
|  | #### OR |  |
| a) | Design a class A type power amplifier circuit using transformer coupling to deliver 8V rms to a load having resistance of 8. Consider Vcc = 18V. | (8) |
| b) | Write a short note on fixed bias circuit. | (4) |
| Q.No.6. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Give the design procedure for UJT relaxation oscillator. |  |
| b) | Give the design steps for astable multivibrator using transistor. |  |
| c) | Write a note on PUT relaxation Oscillator. |  |
|  | ### DATA |  |
| 1) | Transistor BC 147 B (npn) |  |
|  | VCEO= 45V hfe(min)=240 hfe(max)=500 Icmax=100mA hfe(typ)=330 PDmax=0.25W hie=4.5K hoe=30mhos hre=2x10-4 hFE(min)=200, hFE(typ)=290 hFE(max)=450. |  |
| 2) | UJT 2N2646: VBBmax = 35V Vv=1.8V Iv=4mA Ip = 5A min=0.57 typ=0.65 max=0.75 RBBmin=4.7k minimum supply voltage for UJT trigger circuit for different valves of capacitors. C(µf) : 0.02 0.05 0.1 0.2 0.5 0.7 1.0 VBB(volts): 34 24 16 11.5 11 10.5 10 |  |

***********