## BOARD OF TECHNICAL EDUCATION

PORVORIM-GOA

April, 2012 Examinations

Programme: Production Engg.

Subject: Engineering Metallurgy(4073)

# Time Duration: 3 Hrs.                                               Max. Marks:  75

Instructions: 1) All Questions are compulsory.

2) Figures to the right indicate full marks.

3) Assume suitable additional data if required.

| Q.No.1. | Answer any five of the following Questions: | 5 x 3 = 15 |
| --- | --- | --- |
| a) | What do you mean by Eutectics and Eutectoids to the fundamental structure of materials? |  |
| b) | Explain hardness and brittleness of steel. |  |
| c) | What is the atomic arrangement in common metallic materials? |  |
| d) | What are the mechanical properties of alloy steel? |  |
| e) | State the properties and applications of vanadium steels. |  |
| f) | Differentiate between flame hardening and induction hardening. |  |
| g) | Explain the principle of surface hardening of steels by nitriding process. |  |
| Q.No.2. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Explain in brief the properties and applications of nickel zinc and its alloys. |  |
| b) | What are the stages involved in powder metallurgy? Explain the process of manufacturing of cemented carbide tools and ceramics tools. |  |
| c) | Explain with a neat sketch the magnetic particle non destructutive testing process and state its applications. |  |
| Q.No.3. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | What are the important alloys of Aluminium? Give their properties and applications. |  |
| b) | Classify the steel on the basic of carbon content, and explain the effect of heat treatment on its mechanical properties. |  |
| c) | Why a hardened steel is tempered? Explain the methods and effects of tempering. |  |
| Q.No.4. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | What is surface hardening of steels? Explain in brief any one method of surface hardening process. |  |
| b) | Give a typical composition of the following alloy steels: - Molybdenum Steels - Manganese Steels - Boron Steels. |  |
| c) | What is cast iron? Name the types of cast iron and explain in brief any one type of cast iron. |  |
| Q.No.5. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Explain briefly the process involved in the production of metal powders sintering products of power metallurgy. |  |
| b) | What is Annealing of steel? Draw a part of iron carbon equilibrium diagram showing the temperature range for Annealing operation. |  |
| c) | State the various quenching media in the order of quenching speed, and explain their effect on the structure and properties of steel. |  |

………2/-

-2-

| Q.No.6. | Write short notes an any four of the following: | 4 x 3 = 12 |
| --- | --- | --- |
| a) | Titanium bearing metals |  |
| b) | Eddy current testing methods |  |
| c) | Application of TTT curves |  |
| d) | Tungsten steels |  |
| e) | Modular cast iron and alloy cast iron |  |
| f) | White cast iron |  |

***********