## BOARD OF TECHNICAL EDUCATION

PORVORIM-GOA

April, 2012 Examinations

Programme: Fabrication Technology & Erection Engg.

Subject: Piping Fabrication & Installation(5342)

# Time Duration: 3 Hrs.                                               Max. Marks:  75

Instructions: 1) All Questions are compulsory.

2) Figures to the right indicate full marks.

3) Assume suitable additional data if required.

4) Refer to data provided at the end of question Paper.

| Q.No.1. | Answer any five of the following Questions: | 5 x 3 = 15 |
| --- | --- | --- |
| a) | Draw a sketch and explain the hydraulic hand operated means used in pipe bending. |  |
| b) | In hydro testing, what is the pressure used for testing? What is the use of this test? |  |
| c) | Write three differences in between lines carrying steam and those carrying water at room temperature layout. |  |
| d) | Draw a sketch and explain ‘Nozzle Orientation’ & ‘Equipment Orientation’ |  |
| e) | What is the function of each of the following fittings? i)Union coupling ii)Tee iii)Cap. |  |
| f) | What formula can be generally used to find pipe thickness? Explain the terms involved. |  |
| g) | How can elevation or height of a pipeline that which is horizontal be shown in orthographic drawings? |  |
| Q.No.2. | Design a pipe of material ASTM A106 GB to fabricate a pipeline to conduct a fluid at 200C, delivery pressure 6.5 Kg/cm2 and discharge of 0.273 m3/min. The total length of the pipeline is 7250 ft and in its length, 2 Globe valves, 1 gate valve, 1 safety valve, 2 unions and 8 short radius 90 Elbows are present. | (12) |
| Q.No.3. | Answer the following Questions: |  |
| a) | Draw the Isometric view and developed view of the portion of the pipeline whose plan is shown in Fig.1, Q. 3(a) | (8) |
| b) | Make a Bill of material of the pipeline shown in Fig.1. Q.3.(a), Neglect weight of pipeline & all fittings. | (4) |
| Q.No.4. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Enlist six different points to be taken care in making layout of pipeline in an enclosed space where movement of people is involved and to take care of operation of valves, etc. |  |
| b) | Assuming suitable data, draw a P&ID of your own choice having symbolic representation generally used for: - Pressure, temperature & other gauges - Tank/heat exchangers/ Processing vessels - Pipelines and insulated pipelines - Pumps/condensers - Hot well or sump - Any other equipment involved in the process. |  |
| c) | For a 4-piece fabricated pipe bend having 100 change in direction, draw the development of 2nd pipe piece for end profile. Also calculate the minimum length of pipe required to fabricate the pipe bend if O.D. = 60mm and centre line radius is 100mm. |  |

………2/-

-2-

| Q.No.5. | Answer any two of the following Questions: | 2 x 6 = 12 |
| --- | --- | --- |
| a) | Explain the procedure used to apply insulation in pipelines. |  |
| b) | Write a note on selection of welding rods for ferrous pipeline welding. |  |
| c) | Explain any one method of pre-heating and post weld heat treatment each. |  |
| Q.No.6. | Write short notes on any four: | 4 x 3 = 12 |
| i) | Pipeline supports to be used on floors |  |
| ii) | Piping material inspection |  |
| iii) | Corrosion consideration in design of pipelines |  |
| iv) | Bypass arrangement used for pumps, filters, etc. |  |
| v) | Standards for measurement of pipe insulation |  |
| vi) | Major certifying bodies engaged in piping work in India. |  |