## BOARD OF TECHNICAL EDUCATION

PORVORIM-GOA

April, 2012 Examinations

Programme: Common

Subject: Elements of Electrical Engineering(3001)

# Time Duration: 3 Hrs.                                               Max. Marks:  75

Instructions: 1) All Questions are compulsory.

2) Figures to the right indicate full marks.

3) Assume suitable additional data if required.

| Q.No.1. | Answer any five of the following Questions: | 5 x 3 = 15 |
| --- | --- | --- |
| a) | Explain the function of the No-Volt release in a four point starter. |  |
| b) | What will be the effect if a 220V/100V transformer is connected to a 220V D.C. supply? |  |
| c) | Explain the importance of equipment earthing. |  |
| d) | What protection is provided to electrical circuits by MCBs? |  |
| e) | List out any 3 electrical specifications of three phase induction motors. |  |
| f) | Explain the need for placing the fuse in the phase wire. |  |
| Q.No.2. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Mention any three advantages and any three disadvantages of overhead distribution system. |  |
| b) | With a neat labelled diagram, explain the construction of a 3½ core cable. |  |
| c) | List out the specifications for the procurement of PVC cables. |  |
| Q.No.3. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Explain conduit wiring with a neat sketch. |  |
| b) | Explain any 6 I.E. rules relevant to domestic wiring. |  |
| c) | Explain pipe earthing with a neat diagram. |  |
| Q.No.4. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Explain the construction of the stator and rotor of a three phase Squirrel cage induction motor. |  |
| b) | Give any six points of comparison between squirrel cage and slip ring induction motors. |  |
| c) | Write a short note on any one of the following: - Universal Motor - AC series Motor - Permanent capacitor induction motor |  |
| Q.No.5. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Explain the principle of operation of a dc motor. Why do dc motors need the use of a starter? |  |
| b) | With a neat sketch explain the construction and operation of any one type of a dc motor starter. |  |
| c) | Give any 3 points of comparison between core type and shell type transformer. |  |
| Q.No.6. | Write short notes on any three of the following: | 3 x 4 = 12 |
| a) | ELCB |  |
| b) | Types of fuses |  |
| c) | Thermal relays |  |
| d) | Application of transformers. |  |

***********