## BOARD OF TECHNICAL EDUCATION

PORVORIM-GOA

April, 2012 Examinations

Programme: Electronics Engg./EC/E/E&EE

Subject: Electronic Instrumentation(4141)

# Time Duration: 3 Hrs.                                                Max. Marks: 75

Instructions: 1) All Questions are compulsory.

2) Figures to the right indicate full marks.

3) Assume suitable additional data if required.

| Q.No.1. | Answer any five of the following Questions: | 5 x 3 = 15 |
| --- | --- | --- |
| a) | State three points of difference between: - RTD and thermistors - Active and passive transducer. |  |
| b) | Name: - three methods of displacement measurement - 3 methods of level measurement. |  |
| c) | Name: - 3 photo transducers - 3 resistors. |  |
| d) | Name: - Two digital transducers - Two inductive type transducers - Two force summing devices. |  |
| e) | Draw block diagram of final control operation and explain the blocks. |  |
| f) | What are the essential characteristics in an ideal instrumentation amplifier? |  |
| Q.No.2. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | How can potentiometer be used to measure displacement? Explain with figure & expression. Explain principle of strain gauge. Define gauge factor. State two advantages of foil type strain gauge over wire type. |  |
| b) | Explain principle, construction & principle of operation of LVDT, with neat figure. What is residual voltage? What are its causes? |  |
| c) | With neat figures, explain the different ways in which capacitive transducers attain variation in capacitance. State applications of capacitive transducers. |  |
| Q.No.3. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | State six advantages of electrical transducers. What is seebeck effect? In which transducer is it utilised? Explain in detail. |  |
| b) | State six applications of phototransducers. Briefly explain, operation of i) LDR ii) Photovoltaic cell. |  |
| c) | Explain: - Turbine flow meter - Magnetic flow meter. |  |

………2/-

-2-

| Q.No.4. | Answer any two of the following Questions: | 2 x 6 = 12 |
| --- | --- | --- |
| a) | Explain: - Two methods of angular velocity measurement - Two methods of pressure measurement. |  |
| b) | Explain: - Two methods of vibration measurement - One restriction type and one obstruction type method of flow measurement. |  |
| c) | Write note on: - Stroboscope and its uses - Thickness measurement. |  |
| Q.No.5. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Draw block diagram of signal conditioning used with any transducer and explain its features and operation. |  |
| b) | Explain features of data acquisitions system with help of block diagram. |  |
| c) | What are the features of a data logger system? Explain with block diagram. |  |
| Q.No.6. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Classify various continuous controller modes. Explain any one in detail, with mode equation. Which mode is most preferred and why? |  |
| b) | Explain: - Two actuators - Control valves. |  |
| c) | Write a note on ‘DDC and its features’. |  |

***********