## BOARD OF TECHNICAL EDUCATION

PORVORIM-GOA

April, 2012 Examinations

Programme: Shipbuilding Engg.

Subject: Ship Structural Drawing(4341)

# Time Duration: 3 Hrs.                                                Max. Marks: 75

Instructions: 1) Q.No.1 is compulsory. Attempt any 5 from

the remaining Questions.

2) Figures to the right indicate full marks.

3) Assume suitable additional data if required.

| Q.No.1. | Write short notes on any three of the following: | 3 x 5 = 15 |
| --- | --- | --- |
| a) | Still water bending moment |  |
| b) | Section Modulus |  |
| c) | Load Curve |  |
| d) | Keel Plating |  |
| Q.No.2. | Sketch the midship section of an oil tanker showing all structural details. Name the following members. | (12) |
| i) | Longitudinal Bulkhead ii) Longitudinal iii) Girder |  |
| iv) | Bilge plate. |  |
| Q.No.3. | A Box shaped vessel 100 meters long, floats on an even keel displacing 2000 tonnes. The mass of the vessel alone is 1000 tonnes evenly distributed and she is loaded at each end for a length of 25 meters with 500 tonnes of cargo, also evenly distributed. Sketch the curve of loads, shearing force and bending moments. Also state the maximum shearing force and bending moment and state where these occur. | (12) |
| Q.No.4. | Answer the following: |  |
| a) | Explain the procedure of drawing shell expansion diagram. | (6) |
| b) | Detail the importance of shell expansion in shipyard while constructing a ship. | (6) |
| Q.No.5. | Determine the hull girder section modulus of the midship section given in sketch below. | (12) |
| Q.No.6. | Explain in brief the procedure to determine the scantlings of hull structure | (12) |
| Q.No.7. | Answer the following: |  |
| a) | Explain the load curve, S.F. diagrams and B.M. diagrams of a ship structure. | (6) |
| b) | Explain the method of drawing the load curve, S.F. diagram and B.M. diagram with mathematical formulations. | (6) |