## BOARD OF TECHNICAL EDUCATION

PORVORIM-GOA

April, 2012 Examinations

Programme: Electronics Engg./E/EI/COMP/EC/MED/E&EE

Subject: Basic Electronics(4132)

# Time Duration: 3 Hrs.                                                Max. Marks: 75

Instructions: 1) All Questions are compulsory.

2) Figures to the right indicate full marks.

3) Assume suitable additional data if required.

| Q.No.1. | Answer any five of the following Questions: | 5 x 3 = 15 |
| --- | --- | --- |
| a) | Explain the difference between conductors, insulators and semiconductors using the energy band diagrams. |  |
| b) | Draw the output voltage waveform of a halt wave rectifier and then show the effect of connecting a capacitor across the load resistance on the waveform. |  |
| c) | What is ICBO? Explain with a neat diagram of transistor. What is the typical range of its value? |  |
| d) | Explain the need for biasing of a transistor. |  |
| e) | Draw the symbol and plate characteristics of a Pentode tube. |  |
| f) | Enlist few advantages of JFET |  |
| Q.No.2. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Explain the formation of a N-type semiconductor. Enlist the N-type impurities and P-type impurities. |  |
| b) | Explain the operation of halt wave rectifier with a neat diagram. |  |
| c) | In a center-tap full wave rectifier, the load resistance RL= 1 K. Each diode has a forward bias dynamic resistance rd of 10. The voltage across half the secondary winding is (230 sin 314t). Find - The peak value of current - The dc value of current - The rms value of current. |  |
| Q.No.3. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | A 24V, 600mw Zener diode is to be used for providing a 24V stabilized supply to a variable load. If input voltage is 32V, calculate the series resistance Rs required. |  |
| b) | Define Reverse leakage current in common base configuration. Assume a transistor has dc of 0.98 and ICBO of 1A. Calculate its collector and base currents when IE=1mA. |  |
| c) | Draw and explain CE configuration of a transistor. Derive the relation between output current, input current and leakage current for the same. |  |
| Q.No.4. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Explain how stabilization of operating point is achieved when one end of the base resistor RB is connected to the collector terminal instead of the dc supply. |  |
| b) | Describe how thermal runway occurs. How can it be prevented from occurring? |  |
| c) | Explain the voltage-divider biasing circuit. |  |

………2/-

-2-

| Q.No.5. | Answer any two of the following Questions: | 2 x 6 = 12 |
| --- | --- | --- |
| a) | Explain the construction and working of a vacuum diode. |  |
| b) | Explain the working of N-channel JFET. |  |
| c) | i)Draw the symbol and structure of enhancement and depletion type of MOSFET. ii)State two advantages of MOSFET over JFET. |  |
| Q.No.6. | Write short notes on any two of the following: | 2 x 6 = 12 |
| a) | Zener diode as a regulator |  |
| b) | Collector-to-base biasing of a transistor |  |
| c) | Vacuum triode |  |

***********