## BOARD OF TECHNICAL EDUCATION

PORVORIM-GOA

April, 2013 Examinations

Programme: Automobile Engg.

Subject: Garage Equipments(4056)

# Time Duration: 3 Hrs.                                               Max. Marks:  75

Instructions: 1) All Questions are compulsory.

2) Figures to the right indicate full marks.

3) Assume suitable additional data if required.

| Q.No.1. | Answer any five: | 5 x 3 = 15 |
| --- | --- | --- |
| a) | Sketch a compression gauge and explain its working and uses. What units are marked on the gauge? |  |
| b) | Describe the tube repair kit. |  |
| c) | Explain one type of bearing noise tester. |  |
| d) | Describe one instrument for measuring engine cylinder pressure working on piezo-electric principle. |  |
| e) | With neat sketches describe three types of hammers. |  |
| f) | Explain the spark plug cleaner. What gap should be set for the plug? |  |
| Q.No.2. | Answer any two: | 2 x 6 = 12 |
| a) | Explain the construction and working of a grease gun with a neatly labelled figure. |  |
| b) | Sketch the hydraulic and mechanical jacks and distinguish between them. |  |
| c) | How are piston rings assembled on the piston? Explain with sketch one instrument. |  |
| Q.No.3. | Answer any two: | 2 x 6 = 12 |
| a) | Explain the uses of surface plate and height gauge with the help of neat sketches. |  |
| b) | Describe a bevel protractor used in a garage and list its various uses. |  |
| c) | Explain the combination set and pitch gauge with neat sketches. |  |
| Q.No.4. | Answer any two: | 2 x 6 = 12 |
| a) | Show a reciprocating air compressor along with a service line. Give its specifications and uses. |  |
| b) | Explain using neat sketches, how cylinder refacing is done. |  |
| c) | With neat sketches explain the uses of flexible shaft grinder, sander and polisher. |  |
| Q.No.5. | Answer any two: | 2 x 6 = 12 |
| a) | How will you test the efficiency of vehicle brakes? Explain one method. |  |
| b) | Draw and explain the Hartridge Smokemeter. |  |
| c) | Sketch wheel alignment gauge and describe its working and use. |  |
| Q.No.6. | Answer any two: | 2 x 6 = 12 |
| a) | Sketch Taps and Dies and give their uses and specifications. |  |
| b) | Give sketches of open end spanner, scraper and snip & state their uses. |  |
| c) | Describe the battery testing equipment. How to know the state of the battery using this? |  |

***********