## BOARD OF TECHNICAL EDUCATION

PORVORIM-GOA

April, 2013 Examinations

Programme: Electronics Engineering/EC/MED/E&EE

Subject: Analog Electronics(4140)

# Time Duration: 3 Hrs.                                               Max. Marks:  75

Instructions: 1) All Questions are compulsory.

2) Figures to the right indicate full marks.

3) Assume suitable additional data if required.

| Q.No.1. | Answer any five of the following Questions: | 5 x 3 = 15 |
| --- | --- | --- |
| a) | List ideal characteristics of an op amp. |  |
| b) | Explain why rectifiers using op amp are called as precision rectifiers. |  |
| c) | Explain the significance of order of active filter. |  |
| d) | Explain the role of VCO in PLL. |  |
| e) | Explain the concept of amplitude scaling. |  |
| f) | Define input offset current, and input bias current of an op amp. |  |
| g) | Define UTP & LTP in a Schmitt trigger. |  |
| Q.No.2. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Explain averaging and summing amplifier using non-inverting configuration. |  |
| b) | The amplifier having following parameters is connected as inverting amplifier with R1=470Ω, and Rf =4.7KΩ, A=2,00,000, Ri = 2MΩ, Ro=75 Ω, fo=10Hz, supply voltage = + 12 V, output voltage swing = + 10V. Calculate the values of Af, RiF, ROF, fF and VOOT. |  |
| c) | With the help of neat diagram explain the operation of instrumentation amplifier. |  |
| Q.No.3. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | With the help of neat diagram and waveforms explain the operation of Astable Multivibrator using op amp. |  |
| b) | Explain the operation of Quadrature oscillator. |  |
| c) | With a neat circuit diagram and waveforms explain the working of RC phase shift oscillator. |  |
| Q.No.4. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Explain Butterworth low pass filter. |  |
| b) | Design a second order high pass filter having lower cut off frequency of 2KHz. |  |
| c) | Draw a neat diagram, explain the working of sample & hold circuit. |  |
| Q.No.5. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Draw a block diagram of IC 565 and explain the same. |  |
| b) | Explain the capture and phase lock process in PLL. Define capture range and lock range in PLL. |  |
| c) | Explain with neat circuit diagram working of zero crossing detector. |  |
| Q.No.6. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | What do you understand by line and load regulation? Draw and explain the basic circuit of IC voltage regulator. |  |
| b) | Draw a diagram and explain how you can construct a voltage regulator using IC 723 to generate output between 7 to 37V. Derive the expression for the output. |  |
| c) | Explain the concept of solution of differential equations using analog computer. |  |

***********