## BOARD OF TECHNICAL EDUCATION

PORVORIM-GOA

April,2013 Examinations

Programme: Electronics & Instrumentation Engg.

Subject: Machine Technology(4217)

# Time Duration: 3 Hrs.                                           Max. Marks: 75

Instructions: 1) All Questions are compulsory.

2) Figures to the right indicate full marks.

3) Assume suitable additional data if required.

| Q.No.1. | Answer any five of the following Questions: | 5 x 3 = 15 |
| --- | --- | --- |
| a) | Differentiate between “Machine & Structure”. |  |
| b) | Explain the effect of Slip of belt on the Velocity ratio of the belt drive. |  |
| c) | Give the exact placement of “Superheater” in the boiler and state the need for this device. |  |
| d) | Sketch and explain the “Screw Pair”. |  |
| e) | Discuss the salient features of the modern surface condenser and the advantage drawn due to this. |  |
| f) | Select and justify the type of pump you would use under following circumstances: (1)Very high flow rate of fluid (discharge) required (2)Very high suction and discharge head. |  |
| Q.No.2. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Enumerate the inversions of a double slider crank mechanism with examples. Explain any one in detail. |  |
| b) | What are gear trains? Explain the various types of gear trains. |  |
| c) | Two shafts with centre to centre distance between them as 3m are having two pulleys with radii equal to 240 mm & 320 mm respectively. Find the length of the belt required for the open belt drive. |  |
| Q.No.3. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Explain construction and working of a Babcock & Wilcox boiler with a neat sketch. |  |
| b) | What is De Laval nozzle? Also state the principles of a turbine. |  |
| c) | State the difference between mountings and accessories in a steam generator. |  |
| Q.No.4. | Write brief notes on any three: | 3 x 4 = 12 |
| a) | Pressure Compounded Turbine |  |
| b) | Flexible Coupling |  |
| c) | Application of Roots Blower |  |
| d) | Law of Gearing |  |
| Q.No.5. | Answer any three of the following Questions: | 3 x 4 = 12 |
| a) | What are Boiler Accessories? Explain with a sketch an Economiser. |  |
| b) | With a neat sketch explain the working of a Centrifugal Compressor. |  |
| c) | Describe with a neat sketch Low level Jet Condenser. |  |
| d) | Briefly explain reed type valve, plate type valves and combined Valves in air compressors. |  |

………2/-

-2-

| Q.No.6. | Write brief notes on any three: | 3 x 4 = 12 |
| --- | --- | --- |
| a) | Water ring Vacuum pump |  |
| b) | Oldham’s Coupling |  |
| c) | Need for a Condenser |  |
| d) | Trouble shooting in Reciprocating pumps |  |
| e) | Types of Chain drives |  |

***********