## BOARD OF TECHNICAL EDUCATION

PORVORIM-GOA

April,2013 Examinations

Programme: Mining Engg.

Subject: Open Cast Mining Machinery(4299)

# Time Duration: 3 Hrs.                                           Max. Marks: 75

Instructions: 1) All Questions are compulsory.

2) Figures to the right indicate full marks.

3) Assume suitable additional data if required.

| Q.No.1. | Answer the following Questions: |  |
| --- | --- | --- |
| a) | Compare rope shovel with hydraulic shovel. | (4) |
| b) | Explain booster belt conveyor. | (4) |
| c) | What are the criteria to be followed during the selection of a dozer? | (4) |
| d) | Enumerate factors considered in selection of pumps. | (3) |
| Q.No.2. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Write merits and demerits of front-end loader as compared to shovel. |  |
| b) | What are the methods employed for deflecting a borehole? |  |
| c) | Sketch and explain walking mechanism of dragline. |  |
| Q.No.3. | Answer any three of the following Questions: | 3 x 4 = 12 |
| a) | Explain DTH (Down the hole drill). |  |
| b) | Enumerate technical specification of ripper. |  |
| c) | What are the different types of bits used in rotary drilling? |  |
| d) | Sketch and explain swing mechanism of a shovel. |  |
| Q.No.4. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Explain hydraulic circuit of a dumper. |  |
| b) | Explain the purpose of belt take up system. |  |
| c) | Explain cable belt conveyer. |  |
| Q.No.5. | Answer any three of the following Questions: | 3 x 4 = 12 |
| a) | Explain procedure for starting and stoping of pumps including priming. |  |
| b) | Explain how head is developed in a turbine pump. |  |
| c) | Explain the head-volume curve for turbine pump. |  |
| d) | What is an axial thrust? What are the different methods of countering axial thrust? |  |
| Q.No.6. | Answer any three of the following Questions: | 3 x 4 = 12 |
| a) | Write briefly on purpose of clamshell and spreader. |  |
| b) | Sketch and label hydraulic circuit of shovel bucket. |  |
| c) | What are the requirements in the suction pipe of a turbine pump? |  |
| d) | Describe the components of a drill machine and their purpose. |  |

***********