## BOARD OF TECHNICAL EDUCATION

PORVORIM-GOA

April, 2013 Examinations

Programme: Architectural Engg.

Subject: Building Construction-III(4524)

# Time Duration: 3 Hrs.                                               Max. Marks:  75

Instructions: 1) All Questions are compulsory.

2) Figures to the right indicate full marks.

3) Assume suitable additional data if required.

| Q.No.1. | Answer the following Questions: | 5 x 3 = 15 |
| --- | --- | --- |
| a) | Define the tem ‘footing’. |  |
| b) | Sketch a cross–section through a R.C.C. beam showing reinforcement details. |  |
| c) | State two types of dome structures. |  |
| d) | State three recommended sizes of doors. |  |
| e) | List three factors that affect the choice of materials for floors. |  |
| Q.No.2.a) | Answer any one of the following Questions: | (3) |
| i) | State the type of buildings for which domes can be recommended as roofing option. |  |
| ii) | What are the advantages of shell structures? |  |
| b) | State the conditions under which an eccentric footing is provided. | (3) |
| c) | Answer any one of the following | (6) |
| i) | Elaborate on factors that decide the size of foundation in buildings. |  |
| ii) | Draw elaborate sketches to explain reinforcement details for a R.C.C. combined footing. |  |
| Q.No.3.a) | Answer the following Questions: |  |
| i) | Why is damp resistance a necessary requirement of floors? | (3) |
| ii) | Draw three types of steel doors frames. | (3) |
| b) | Elaborate on the considerations a planner has to make while locating doors and windows in any building. | (6) |
| Q.No.4.a) | Answer any one of the following Questions: | (6) |
| i) | Write a detailed note on timber floor. |  |
| ii) | Suggest the type of flooring for a sound recording studio considering sound insulation aspect. Draw details of the floor. |  |
| b) | Answer any one of the following Questions: | (6) |
| i) | What are the requirements of good roof? |  |
| ii) | Draw line sketches of four types of steel trusses. |  |
| Q.No.5.a) | Answer any one of the following Questions: | (6) |
| i) | Write a short note on Hinges used in doors and windows. |  |
| ii) | Write a brief note on factors that decide the shape and size of window. |  |
| b) | Answer any one of the following Questions: | (6) |
| i) | Sketch different types of bolts and screws used for fixing A.C. roof sheet. |  |
| ii) | Sketch various details of fixing A.C. sheet to Purlins. |  |
| Q.No.6. | Draw and explain procedure of fixing Mangalore tiles over R.C.C. sloping roof. Make suitable assumptions. | (12) |

***********