## BOARD OF TECHNICAL EDUCATION

PORVORIM-GOA

April, 2013 Examinations

Programme: Mechanical Engg.

Subject: Power Engineering-I(4037)

# Time Duration: 3 Hrs.                                               Max. Marks:  75

Instructions: 1) All Questions are compulsory.

2) Figures to the right indicate full marks.

3) Assume suitable additional data if required.

4) Use of Steam Tables is permissible.

| Q.No.1. | Answer any five of the following Questions: | 5 x 3 = 15 |
| --- | --- | --- |
| a) | Briefly explain Stoichiometric air-fuel ratio. |  |
| b) | State and explain zeroth law of thermodynamics. |  |
| c) | Explain Geothermal energy harnessing. |  |
| d) | Define Dryness fraction & explain briefly. |  |
| e) | Define Boiler Drought. List different types of Boiler droughts. |  |
| f) | Classify Air Compressors. |  |
| Q.No.2. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | List the different Renewable sources of energy. Give atleast 5 advantages of Renewable over Non-Renewable sources, with explanation. |  |
| b) | What is an isobaric process? With P-V diagram derive (i) P-V-T relation (ii) work done (iii) change in Internal energy for isobaric process. |  |
| c) | A fuel has following percentage composition by mass: CO2 13.3% CO 0.95% O2 8.35% N2 77.4% Convert this into Volumetric analysis. |  |
| Q.No.3. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | With neat sketch explain “Bomb Colorimeter”. |  |
| b) | Explain with diagram any one Water-tube Boiler. |  |
| c) | - Determine volume of 1kg of superheated steam at a pressure of 20 bar and temperature of 300C. - Calculate Internal energy of 1kg of steam at pressure 10 bar, when steam is 0.9 dry. |  |
| Q.No.4. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | With neat P-V and T-s diagram, explain Carnot cycle. |  |
| b) | With neat sketch, explain single stage Reciprocating Air Compressor. |  |
| c) | A mass of 2.25 kg of Nitrogen occupying 1.5m3 is heated from 25C to 200C at constant volume. Calculate initial and final pressure of gas. (Take universal gas constant Ru= 8314 J/Kg molK and Molecular mass of Nitrogen(M)= 28) |  |
| Q.No.5. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | With neat sketch explain function, construction and working of Lever Safety Valve of Boiler. |  |
| b) | Define clearance volume in Compressor. Mention advantages of multi staging in Air compressors with simple schematic diagram. |  |

………2/-

-2-

| c) | A steam power plant is supplied with dry saturated steam at a pressure of 12 Bar and exhausts into a condenser at 1.0 bar. Calculate Rankine cycle efficiency by using steam tables. Also find for the same data, Carnot cycle efficiency. |  |
| --- | --- | --- |
| Q.No.6. | Write short notes on any four: | 4 x 3 = 12 |
| a) | Calorific value of fuels |  |
| b) | Mollier chart |  |
| c) | Economiser |  |
| d) | Entropy |  |
| e) | Compressed Air Motor (State applications) |  |
| f) | Advantages of Superheating |  |

`                                   ***********