## BOARD OF TECHNICAL EDUCATION

PORVORIM-GOA

April,2013 Examinations

Programme: Common

Subject: Elements of Electrical Engineering(3001)

# Time Duration: 3 Hrs.                                           Max. Marks: 75

Instructions: 1) All Questions are compulsory.

2) Figures to the right indicate full marks.

3) Assume suitable additional data if required.

| Q.No.1. | Answer any five of the following Questions: | 5 x 3 = 15 |
| --- | --- | --- |
| a) | With circuit diagram explain any one method to reverse the direction of rotation of a dc series motor. |  |
| b) | Explain how the eddy current and hysteresis losses are minimized in a transformer. |  |
| c) | Give any three methods of reducing earth resistance. |  |
| d) | Explain the function of a fuse in an electrical circuit. |  |
| e) | What type of motors are used for the following: i) Ceiling fan ii) Automobile starter motor iii) Food mixer. |  |
| f) | Draw a neat sketch of a 3-point electrical wall socket (outlet) and label the various points. |  |
| Q.No.2. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | With a neat sketch explain any one method of laying underground cable. |  |
| b) | With a neat sketch explain the construction of a 3-core PVC insulated armoured cable. |  |
| c) | List the specifications for the procurement of PVC cables. |  |
| Q.No.3. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Explain any one type of test performed on a wiring installation that requires the use of a Meggar. |  |
| b) | State any six I.E. rules relevant to wiring installation. |  |
| c) | With a neat sketch explain plate earthing. |  |
| Q.No.4. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Explain with a neat diagram, the construction and operation of any one type of three phase induction motor starter. |  |
| b) | Name any 3 different types of single phase induction motor and give any two applications of each. |  |
| c) | Write a note on any one: - Squirrel Cage induction motor - AC Commutator motors. |  |
| Q.No.5. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Name any six different parts of a dc motor and give one function of each of them. |  |
| b) | Draw and explain the Speed-Torque characteristics of a dc series and a dc shunt motor. |  |
| c) | For a transformer define the following: i)Efficiency ii)Regulation iii)Primary and secondary winding. |  |
| Q.No.6. | Write notes on any two: | 2 x 6 = 12 |
| a) | MCB |  |
| b) | HRC fuse |  |
| c) | Relays |  |
| d) | Power losses in a transformer |  |

***********