## BOARD OF TECHNICAL EDUCATION

PORVORIM-GOA

April,2013 Examinations

Programme: Shipbuilding Engg.

Subject: Basic Ship Theory-I(4331)

# Time Duration: 3 Hrs.                                           Max. Marks: 75

Instructions: 1) Questions No. 1 is compulsory. Answer any 5 from the remaining

Questions.

2) Figures to the right indicate full marks.

3) Assume suitable additional data if required.

| Q.No.1. | Write short note on any five of the following: | 5 x 3 = 15 |
| --- | --- | --- |
| a) | Container ships |  |
| b) | Bilge radius and rise of floor |  |
| c) | Effect of lifting of weight by crane on KG of a ship |  |
| d) | Table of offsets |  |
| e) | Trapezoidal rule |  |
| f) | Righting moment |  |
| g) | TPC |  |
| Q.No.2. | Answer the following: |  |
| a) | A ship has 300 tonne of cargo in the hold, 24m forward of midships. The displacement of the vessel is 6000 tonnes and its centre of gravity is 1.2m forward of midships. Find the new position of the centre of gravity if this cargo is moved to an after hold, 40m from midships. | (09) |
| b) | Explain with sketch a body plan. | (03) |
| Q.No.3. | The immersed cross sectional areas of a ship 120m long commencing from aft are 2,40,79,100,103,104,104,103,97,58 and 0 m2. Calculate i) Displacement ii) Longitudinal position of the centre of buoyancy. | (12) |
| Q.No.4. | Answer the following: |  |
| a) | The waterplane areas of a ship at 1.25m intervals of draught commencing from 7.5m waterline are | (08) |
| Draft Waterplane Areas (m2); 7.5; 8.75; 10.00; 11.25; 12.50. 1845; 1690; 1535; 1355; 1200. |
|  | Draw the curve of TPC and determine the mass to be added to increase the mean draft from 8.10m to 8.30m. |  |
| b) | Explain Block coefficient of a ship. Block co-efficient of Bulk carrier is higher than that of passenger ship. Why? | (04) |
| Q.No.5. | Answer the following: |  |
| a) | Explain stable equilibrium and righting moment with neat sketch. | (06) |
| b) | Draw a neat sketch of GZ curve & explain various parameters of stability determined from it. | (06) |

………2/-

-2-

| Q.No.6. | Given the KN values of a ship at displacement of 40000 tonnes, draw the curves of statical stability, if the KG is at 9.8m. Heel Angle; 5; 10; 15; 20; 30; 45; 60; 75; 90. KN(m); 0.9; 1.92; 3.11; 4.25; 6.30; 8.44; 9.39; 9.29; 8.50. Also find i) Range of stability ii) Angle of vanishing stability iii) Maximum GZ iv) Initial Metacentric height. | (12) |
| --- | --- | --- |
| Q.No.7. | The half ordinates of a waterplane area 180m long are as follows Section; AP; 1; 2; 3; 4; 5; 6. ½ ord; 0; 8.0; 10.5; 12.5; 11.0; 3.0; 1.0. | (12) |
|  | Calculate - TPC - LCF - Second moment of area of waterplane about centerline. |  |

***********