## BOARD OF TECHNICAL EDUCATION

PORVORIM-GOA

April,2013 Examinations

Programme: Mining Engg.

Subject: Underground Mining Machinery(4294)

# Time Duration: 3 Hrs.                                           Max. Marks: 75

Instructions: 1) All Questions are compulsory.

2) Figures to the right indicate full marks.

3) Assume suitable additional data if required.

| Q.No.1. | Answer any five of the following: | 5 x 3 = 15 |
| --- | --- | --- |
| a) | What are the tests conducted to judge the quality of wire rope? |  |
| b) | Write short notes on skips. |  |
| c) | Give the technical specifications of a Jack hammer drill? |  |
| d) | Write short notes on continuous miner. |  |
| e) | What are the different track mounted haulages used in underground mines? |  |
| f) | What are the safety devices used in track mounted haulage roadways? |  |
| Q.No.2. | Answer any two of the following: | 2 x 6 = 12 |
| a) | Explain the construction of flattened strand rope. Give its advantages. |  |
| b) | What are the different equipments installed on a shaft? Give purpose of each. |  |
| c) | Explain the salient features of traverse and turntable layouts. |  |
| Q.No.3. | Answer any two of the following: | 2 x 6 = 12 |
| a) | Explain Koepe winding system with neat sketches. |  |
| b) | Write short notes on (i) Drum winder (ii) Detatching hook. |  |
| c) | 180 T of coal is to be hauled by main direct rope haulage in shift of 6 hr hauling time, where the length of haulage road is 1600 m with a gradient of l in 10. The average rope speed is 7.5 Kmph and set changing time is 2 minutes. Tub capacity is 1.5T. Calculate the number of tubs per train. |  |
| Q.No.4. | Answer any two of the following: | 2 x 6 = 12 |
| a) | Explain the construction & operation of a coal drill. |  |
| b) | Differentiate between SERDS & DERDS? Discuss on the drum of a Shearer. |  |
| c) | Explain the construction & operation of a Side Discharge Loader (SDL). |  |
| Q.No.5. | Answer any two of the following: | 2 x 6 = 12 |
| a) | Describe an endless haulage system. Also explain how the tubs are attached to the haulage rope. |  |
| b) | What are the dangers and hazards that may arise from use of diesel locomotive in underground mines? Name the fittings & safety features required in diesel loco used in underground coal mines. |  |
| c) | A locomotive weighs 5 T and the adhesion to the track is 200 Kg per Ton weight of loco. Assuming rolling friction for loco is 10Kg/ton, calculate (i) Coefficient of adhesion (ii) Maximum tractive effort the loc can exert (iii) Maximum draw bar pull on the level (iv) Maximum draw bar pull up gradient of 1 in 50. | ……2/- |

-2-

| Q.No.6. | Answer any three of the following: | 3 x 4 = 12 |
| --- | --- | --- |
| a) | Explain the process of any one type of rope capping. |  |
| b) | Explain functioning of any one type of mechanical brake used in winding. |  |
| c) | Write short notes on shuttle car. |  |
| d) | What are the essentials of a good haulage track to be used in mines? |  |

***********