## BOARD OF TECHNICAL EDUCATION

PORVORIM-GOA

April, 2013 Examinations

Programme: Electronics Engg./EI/COMP/EC/MED/E&EE

Subject: Digital Electronics-II(4138)

# Time Duration: 3 Hrs.                                               Max. Marks:  75

Instructions: 1) All Questions are compulsory.

2) Figures to the right indicate full marks.

3) Assume suitable additional data if required.

| Q.No.1. | Answer any five: | 5 x 3 = 15 |
| --- | --- | --- |
| a) | Draw the logic diagram of Half Adder and Half subtractor. Write the truth table of each. |  |
| b) | What is a Priority Encoder? Draw the logic diagram for the same. |  |
| c) | Define: Resolution, Accuracy and Conversion for an Analog to Digital Converter. |  |
| d) | List three applications and three advantages of Programmable Logic Devices. |  |
| e) | What are the output voltages caused by each bit in a 3-bit ladder network; if 0=0V and 1=+10V. |  |
| f) | Show the internal organisation of 16 x 4 memory chip. |  |
| g) | State the functions of Multiplexer, Demultiplexer and Decoder. |  |
| h) | List the three requirements that an ideal clock must satisfy. |  |
| Q.No.2. | Answer any two: | 2 x 6 = 12 |
| a) | With a neat diagram and waveforms, explain the operation of IC 555 as a Monostable Multivibrator. |  |
| b) | For a Astable Multivibrator using IC 555, given RA=750Ω. Determine the values for RB and C, to provide a 1 MHZ clock that has a 25 percent duty cycle. |  |
| c) | Draw the following: (i) One Cell Dynamic RAM (ii)TTL Clock circuit. |  |
| Q.No.3. | Answer any two: | 2 x 6 = 12 |
| a) | Draw and explain how a 16 x 8 memory can be constructed using two 16 x 4 memory chips. |  |
| b) | Write short notes on: - PROM - Static RAM. |  |
| c) | Draw the BCD Adder and explain its working with an example. |  |
| Q.No.4. | Answer any two: | 2 x 6 = 12 |
| a) | Draw a 8-bit Adder-Subtractor circuit and explain its operation with an example. |  |
| b) | List the different types of PLDs and explain any one in detail. |  |
| c) | What is a PAL? What are registered PALS and Configurable PALS. |  |

………2/-

-2-

| Q.No.5. | Answer any two: | 2 x 6 = 12 |
| --- | --- | --- |
| a) | With neat diagrams, explain in brief the following - Series Adder - Programmable Array Logic. |  |
| b) | With a neat circuit diagram, explain the operation of IC 7448 decoder driver. |  |
| c) | With a neat logic diagram and truth table, explain the operation of a 1:16 demultiplexer. |  |
| Q.No.6. | Answer any two: | 2 x 6 = 12 |
| a) | With a neat diagram, explain the counter type method of Analog to Digital conversion. State its advantages. |  |
| b) | Draw a 4-bit R-2R ladder network, and explain the process of Digital to Analog conversion with an example. |  |
| c) | For a 6-bit binary ladder network, if 0=0V and 1=+10V, find the output voltage for the following inputs: - (101001)2 - (111011)2 |  |

***********