## BOARD OF TECHNICAL EDUCATION

PORVORIM-GOA

April, 2013 Examinations

Programme: Mechanical Engg./P/A

Subject: Machine Drawing-I(4033)

# Time Duration: 4 Hrs.                                               Max. Marks:  75

Instructions: 1) All Questions are compulsory.

2) Figures to the right indicate full marks.

3) Assume suitable additional data if required.

| Q.No.1. | Attempt any two of the following: | 2 x 6 = 12 |
| --- | --- | --- |
| a) | The front view of a 120mm long line AB measures 70mm and its top view measures 90mm. The mid-point M of the line AB lies at a distance of 60mm from both the planes. Draw the projections of the line. |  |
| b) | Fig.1 shows two views of an object drawn by first angle method of projection. Draw the auxiliary front view in the direction of arrow ‘A’. |  |
| c) | An equilateral triangle ABC of sides 75mm long has its side AB in the VP and inclined at 60º to the HP. Its plane makes an angle of 45º with the VP. Draw its projections. |  |
| Q.No.2. | A cylindrical pipe of 30mm diameter has a similar branch of the same size. The axis of the main pipe is vertical and is intersected by that of the branch at right angles. Develop the surfaces of the two pipes, assuming a suitable length for the two pipes. | (10) |
|  | ### OR |  |
| Q.No.2. | Draw the development of the lateral surface of the part of the cone, the front view of which is shown in fig.2. | (10) |
| Q.No.3. | A right regular pentagonal prism having sides of base equal to 30mm and a height of 70mm stands on its base on the HP such that one of its base edges is parallel to the VP. It is penetrated by a horizontal cylinder, the axis of which is parallel to the VP and 5mm away from that of the prism. The diameter of the cylinder is 40mm. Draw the projections of the solids showing the curves of intersection. | (12) |
|  | ### OR |  |
| Q.No.3. | A vertical cylinder of 60mm diameter and 80mm length is penetrated by a horizontal regular pentagonal prism of sides of base 15mm. The axis of the prism is parallel to the VP and passes through the midpoint of the axis of the cylinder. One vertical face of the prism is parallel to the VP. Draw the projections of the solids showing the curves of intersection. | (12) …….2/- |
|  | -2- |  |
| Q.No.4. | Fig.3 shows the front view and side view of a wall Bracket. Draw according to third angle method of projection - Sectional front view in section AA - Sectional left side view in section CC iii) Top view. | (16) |
| Q.No.5. | Attempt the following: |  |
| a) | With the help of suitable freehand sketches show the following types of sections: | (8) |
|  | - Aligned section - Removed section - Half section - Offset section. |  |
|  | ### OR |  |
| a) | Give the conventional representation for the following: - Right hand single start thread - Helical compression spring with square section - Internal thread - Glass. | (8) |
| b) | Draw using a double line method a 25mm pipe showing the following: - 45º lateral - Reducing socket - Plug. | (9) |
| Q.No.6. | Fig.4 shows a part to be fabricated by welding process. Draw the two views of the part and select suitable types of welds and indicate them through the appropriate weld symbols. | (8) |
|  | ### OR |  |
| Q.No.6. | Show the welding symbols for the following types of welds: - Square butt weld - Single bevel butt weld - Single J butt weld - Fillet weld - Spot weld - Seam weld - Single U parallel weld - Weld all round. | (8) |

…3/-