## BOARD OF TECHNICAL EDUCATION

PORVORIM-GOA

April, 2013 Examinations

Programme: Architectural Engg.

Subject: Structures-I(4537)

# Time Duration: 3 Hrs.                                               Max. Marks:  75

Instructions: 1) All Questions are compulsory.

2) Figures to the right indicate full marks.

3) Assume suitable additional data if required.

| Q.No.1. | Answer any five of the following Questions: | 5 x 3 = 15 |
| --- | --- | --- |
| i) | Explain the characteristics of load bearing structure. |  |
| ii) | Explain the term ‘Principle of transmissibility of forces’. |  |
| iii) | Define centre of gravity and centroid. |  |
| iv) | Sketch any two types of beams based on end condition. |  |
| v) | Explain the terms “grade of concrete” and “characteristics strength of concrete”. |  |
| vi) | Write in brief on Corbelling and its importance. |  |
| Q.No.2. | Answer any two of the following Questions: | 2 x 6 = 12 |
| i) | Give the detailed classification of system of forces. |  |
| ii) | Enlist the various methods of composition of forces & explain Algebraic method. |  |
| iii) | Find the magnitude and resultant of the following system of forces. - 4T – acting North - 8T – acting N-W towards the point - 2T – acting towards East - 6T – acting 60º North of West away from the point. |  |
| Q.No.3. | Answer any two of the following Questions: | 2 x 6 = 12 |
| i) | Enlist the different grades of concrete and their uses. |  |
| ii) | Explain the detailed classification of structures. |  |
| iii) | Differentiate between: - Short column and long column - One-way slab and two way slab - Working stress method and Limit state method of design. |  |
| Q.No.4. | Answer any two of the following Questions: | 2 x 6 = 12 |
| i) | Explain in detail the different types of loads acting on the structure. |  |
| ii) | Draw B.M.D. for the simply supported beam AB of span 6m acted upon by two point loads 50 KN and 60 KN acting at a distance 2m and 4m respectively from left support. |  |
| iii) | A cantilever beam 3m span acted upon by a U.D.L 2KN/m and a point load 20 KN acting at the free end. Draw B.M.D. and S.F.D. |  |
| Q.No.5. | Answer any two of the following Questions: | 2 x 6 = 12 |
| i) | Enlist and explain the causes for development of cracks in concrete. |  |
| ii) | Sketch and explain the following: 1)Arches 2)Shells 3)Domes. |  |
| iii) | Explain any two methods of prevention of cracks. |  |
|  |  | …2/- |
|  | -2- |  |
| Q.No.6. | Write a short note on any four: | 4 x 3 = 12 |
| i) | Special consideration in earthquake resistant construction |  |
| ii) | Moment of forces |  |
| iii) | Settlement of soil |  |
| iv) | Concrete mix design |  |
| v) | Space structures |  |