## BOARD OF TECHNICAL EDUCATION

PORVORIM-GOA

April, 2013 Examinations

Programme: Electronics Engg./EC

Subject: Communication Systems-I(4150)

# Time Duration: 3 Hrs.                                               Max. Marks:  75

Instructions: 1) All Questions are compulsory.

2) Figures to the right indicate full marks.

3) Assume suitable additional data if required.

| Q.No.1. | Answer any five: | 5 x 3 = 15 |
| --- | --- | --- |
| a) | Define frequency modulation. |  |
| b) | What is the principle of AGC? Give one example. |  |
| c) | Define noise in communication systems. |  |
| d) | Give one advantage and disadvantage of SSB over DSB. |  |
| e) | Draw labelled block diagram of stereophonic FM receiver. |  |
| f) | Compare low level and high level transmitter. |  |
| g) | Draw the circuit diagram of diode detector for demodulation of AM signal. |  |
| Q.No.2. | Answer any two: | 2 x 6 = 12 |
| a) | Explain in brief any four internal noises that get generated in communication systems. |  |
| b) | Define AM and derive the expression for AM wave and draw the frequency spectrum of the AM signal. |  |
| c) | A broadcast transmitter radiates 1 KW when the modulation depth is 20%. Calculate the total power when modulation index has been raised to 50%. |  |
| Q.No.3. | Answer any two: | 2 x 6 = 12 |
| a) | Draw the circuit diagram of diode balanced modulator and explain its working. |  |
| b) | With the help of block diagram explain the phase shift method of sideboard suppression. |  |
| c) | i) Compare AM & FM. ii) Differentiate between narrowband and wideband FM. |  |
| Q.No.4. | Answer any two: | 2 x 6 = 12 |
| a) | Explain the working of stereophonic broadcast transmitter. |  |
| b) | Explain pre-emphasis and de-emphasis with the help of circuit diagram and waveform. Why is it used in communication systems? |  |
| c) | Explain the generation of FM by basic reactance modulator. |  |
| Q.No.5. | Answer any two: | 2 x 6 = 12 |
| a) | Explain with the help of circuit diagram and waveform basic diode detector for demodulation of AM signal. |  |
| b) | Write short note on superheterodyne tracking. |  |
| c) | Write short note on Tuned radio frequency receiver. |  |
| Q.No.6. | Write short note on any three: | 3 x 4 = 12 |
| a) | Squelch circuit |  |
| b) | Automatic frequency control |  |
| c) | Block diagram of FM Receiver |  |
| d) | Balanced slope detector |  |

***********