## BOARD OF TECHNICAL EDUCATION

PORVORIM-GOA

April,2013 Examinations

Programme: Automobile Engg.

Subject: Hydraulic Machinery(4053)

# Time Duration: 3 Hrs.                                           Max. Marks: 75

Instructions: 1) All Questions are compulsory.

2) Figures to the right indicate full marks.

3) Assume suitable additional data if required.

| Q.No.1. | Answer any five of the following Questions: | 5 x 3 = 15 |
| --- | --- | --- |
| a) | If a certain liquid has a mass density of 1200Kg/m3, calculate its specific weight and specific gravity. |  |
| b) | Define gauge pressure and absolute pressure. Indicate their position with reference to absolute zero pressure. |  |
| c) | Explain steady-uniform flow with an example. |  |
| d) | State Pascal’s law and give some examples where this principle can be applied. |  |
| e) | What do you mean by ‘Manometric head’ in a centrifugal pump? |  |
| f) | What is a plunger pump? Where would you prefer to use the same? |  |
| g) | Define i) Compressibility ii) Dynamic Viscosity. |  |
| Q.No.2. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Explain with a neat sketch Bourdon tube pressure gauge. |  |
| b) | Describe the construction and working of a single acting reciprocating pump with a neat sketch. |  |
| c) | With the help of a neat sketch explain the working of hydraulic intensifier. |  |
| Q.No.3. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Explain with neat sketches any two types of casings in centrifugal pump. |  |
| b) | A rectangular plate 2m long and 1m wide is immersed vertically in water with its shorter side parallel to the water surface and 3m below it. Find i) Total pressure on the plate and ii) position of centre of pressure. |  |
| c) | Explain the construction and working of vane pump with a neat sketch. |  |
| Q.No.4. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Find the discharge over a triangular notch of angle 60 when the head over the triangular notch is 0.4 m. Assume Cd=0.6. |  |
| b) | A horizontal pipe carries water at the rate of 0.05 m3/s. Its diameter suddenly reduces from 300mm to 100mm. Calculate the loss of head due to sudden contraction. Take coefficient of contraction Cc=0.62. |  |
| c) | A Venturimeter with 150mm diameter at inlet and 100mm at throat is laid with its axis horizontal and is used for measuring the flow of oil of specific gravity 0.8. The mercury differential manometer shows a gauge difference of 200mm. Calculate the discharge. Take coefficient of meter as 0.98. |  |

………2/-

-2-

| Q.No.5. | Answer any two of the following Questions: | 2 x 6 = 12 |
| --- | --- | --- |
| a) | Describe the construction and working of hydraulic Accumulator with a neat sketch. |  |
| b) | What is slip? Under what conditions negative slip occurs in reciprocating pump? |  |
| c) | Explain with a neat sketch the venacontracta and hydraulic coefficients for an orifice. |  |
| Q.No.6. | Write short notes on any three following: | 3 x 4 = 12 |
| a) | Laws of fluid friction |  |
| b) | Priming methods in centrifugal pump |  |
| c) | Barometer |  |
| d) | Limitations of Bernoulli’s theorem |  |

***********