## BOARD OF TECHNICAL EDUCATION

PORVORIM-GOA

April, 2013 Examinations

Programme: Electronics Engg./EC

Subject: Power Electronics(4143)

# Time Duration: 3 Hrs.                                               Max. Marks:  75

Instructions: 1) All Questions are compulsory.

2) Figures to the right indicate full marks.

3) Assume suitable additional data if required.

| Q.No.1. | Answer any five of the following Questions: | 5 x 3 = 15 |
| --- | --- | --- |
| a) | Give two points of distinction between DIAC & TRIAC. |  |
| b) | Give two merits of Servo type a.c. regulator. Also give one of its demerits. |  |
| c) | State the precautions to be taken during parallel operation of diodes. Also draw a neat circuit diagram indicating its operation. |  |
| d) | What is the function of a Snubber circuit? How is it connected across a thyristor? Hence draw a neat circuit diagram. |  |
| e) | Which method is called as False triggering method of triggering of thyristor? and why? |  |
| f) | Define reverse recovery time of diode. How does it influence switching operation of a diode? |  |
| g) | State any two merits & one demerit of saturable core reactor. |  |
| Q.No.2. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Draw the volt ampere characteristics of a LASCR. Also state the method used for its triggering. State the voltage and current rating of LASCR & give two of its applications. |  |
| b) | State the methods used for communication of d.c. operated thyristors & explain any one of such method of your choice with the help of neat circuit diagram. |  |
| c) | Define dv/dt & di/dt for thyristor. Also state the critical limits for dv/dt & di/dt . How can a thyristor be protected against high dv/dt. Explain with the help of neat circuit diagram. |  |
| Q.No.3. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Draw a neat circuit diagram of UJT relaxation oscillator and explain its working in brief with the help of emitter voltage waveform and output voltage waveform. |  |
| b) | Draw SCR transient characteristics and explain it in brief. |  |
| c) | Give the advantage of pulse triggering method for SCR. Also state the methods used for pulse triggering and explain any one of such method of your choice with the help of neat circuit diagram. |  |
| d) | Draw a neat circuit diagram for series operation of power diodes. What precautions is to be taken during such operation? Also give the requirements of such operation. |  |
| Q.No.4. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Draw a neat circuit diagram of single phase full wave bridge controlled rectifier operating resistive load and show its input voltage, output voltage and load current waveform and explain the circuit in brief. |  |
|  |  | …..2/- |
|  | -2- |  |
| b) | With reference to three phase half wave controlled rectifier, draw - Its circuit diagram - Input voltage waveform - Output voltage waveform - Output current waveform. Also state the expression for its output voltage. |  |
| c) | With reference to single phase half wave controlled rectifier operating RL Load. Draw its - Circuit diagram - Supply voltage, output voltage & Output current waveforms. What is the significance of free wheeling diode in this circuit? |  |
| Q.No.5. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | With the help of a neat circuit diagram explain the single phase series inverter in brief. Also draw its load current, output voltage waveform. |  |
| b) | Draw a neat circuit diagram of Morgan Chopper & explain its working in brief with the help of operational diagram. |  |
| c) | Draw a neat circuit diagram of Cycloconverter used to generate output frequency of 1/3rd of its input frequency. Also draw its input voltage, output voltage and output current waveform. Also explain its working in brief. |  |
| Q.No.6. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | With the help of a neat circuit diagram explain the working of Light dimmer circuit using TRIAC. |  |
| b) | How can a.c. power control be achieved using SCR? Hence draw its neat circuit diagram and explain its working in brief. |  |
| c) | Draw the block diagram of SMPS & explain its working in brief. |  |
| d) | Draw a neat circuit diagram of SCR controlled static regulator & explain its working in brief. |  |

***********