## BOARD OF TECHNICAL EDUCATION

PORVORIM-GOA

April, 2013 Examinations

Programme: Mining Engg.

Subject: Surface Mining(4298)

# Time Duration: 3 Hrs.                                               Max. Marks:  75

Instructions: 1) All Questions are compulsory.

2) Figures to the right indicate full marks.

3) Assume suitable additional data if required.

| Q.No.1. | Answer any five of the following Questions: | 5 x 3 = 15 |
| --- | --- | --- |
| a) | Give suitability conditions of opencast mining. |  |
| b) | Compare crawler mounted V/s tyre mounted ripper. |  |
| c) | State suitability conditions of shovel. |  |
| d) | Enumerate advantages and disadvantages of truck transportation system in mining. |  |
| e) | Sketch and give various types of dumps. |  |
| f) | List out machinery used on Goan iron ore mines. |  |
| Q.No.2. | Answer the following Questions: |  |
| a) | Explain various factors affecting selection of opencast mining methods. | (5) |
| b) | Enumerate cycle of operation on open cast mining. | (4) |
| c) | Enumerate factors affecting bench height. | (3) |
| Q.No.3. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Explain methods of grubbing. |  |
| b) | State guidelines for efficient ripping. |  |
| c) | Explain controlled blasting. |  |
| Q.No.4. | Answer the following Questions: | 2 x 6 = 12 |
| a) | Explain working parameters of shovel. |  |
|  | ### OR |  |
| a) | How you will calculate capacity of a shovel and what are the factors affecting it? |  |
| b) | Explain cutting method of bucket wheel excavator (BWE). |  |
| Q.No.5. | Answer the following Questions: | 2 x 6 = 12 |
| a) | Explain haul road geometry. |  |
|  | ### OR |  |
| a) | In an open pit mine a bench of 10m height is being mined by 6m3 shovel to keep the face width 10m. The mining data are as follows: Material swell factor – 1.2 bulk weight of broken material – 2.5 t/m3 bucket fill factor – 0.8 shovel cycle time – 40 seconds working condition – Average The mine works in 2 shifts/day of 8 hours duration each, out of which 7 hours are used for production. Calculate annual production of shovel and face advance per day. |  |

………2/-

-2-

| b) | In one year 2.4 million m3 of over burden is proposed to be removed from a shovel. The other parameters are: Bench height – 14m Expected rate of drilling – 5m/hr Burden- 8m Spacing- 5m Subgrade drilling – Nil Availability of drill = 5000/year Uitilization = 65% of availability Calculate number of drill required for the purpose. |  |
| --- | --- | --- |
| Q.No.6. | Write short notes on any four: | 4 x 3 = 12 |
| a) | Powder factor |  |
| b) | Shovel v/s wheel loader |  |
| c) | Belt conveyor in mines |  |
| d) | Lighting standard in open cast mine |  |
| e) | Need of reclamation in Goan iron ore mines |  |

***********