## BOARD OF TECHNICAL EDUCATION

PORVORIM-GOA

April, 2013 Examinations

Programme: Electrical Engg.

Subject: Distribution & Estimation(4108)

# Time Duration: 3 Hrs.                                               Max. Marks:  75

Instructions: 1) All Questions are compulsory.

2) Figures to the right indicate full marks.

3) Assume suitable additional data if required.

| Q.No.1. | Answer any five of the following Questions: | 5 x 3 = 15 |
| --- | --- | --- |
| a) | What do you mean by feeder and distributor? |  |
| b) | Explain the concept of corrected load and diversity factor. |  |
| c) | Briefly explain the different types of conductor configurations. |  |
| d) | State IE rules applicable to industrial wiring. |  |
| e) | List the methods to improve earth resistance. |  |
| f) | What is cleat wiring ? State its applications. |  |
| Q.No.2. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Derive and compare volume of conductor material required in AC single phase 2 wire system with 3 phase AC three wire 8/m. |  |
| b) | With the help of neat diagram explain radial, ring and intercorrected distribution system. |  |
| c) | A 2 wire distributor 1000m long is loaded as shown in fig(a). B is the mid point. The power factor at the two points refer to voltage at C. The impedance of each line is (0.25+j 0.25). calculate the sending end current and power factor. The voltage at point C is 220V. (see figure on pg.3) |  |
| Q.No.3. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Calculate loop inductance per KM of a single phase transmission line comprising of two parallel conductors 1.25 m apart and 1.5 cm in diameter. Also calculate reactance of line at a frequency=50Hz. |  |
| b) | With a neat diagram explain cage guarding and cradle guarding. |  |
| c) | List different components of distribution lines and explain function of each. |  |
| Q.No.4. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | A 1km long overhead distribution line of 415V, 3 50Hz is to be erected along a straight route from 100 KVA 11/0.43 3KV pole mounting substation. The line is to be laid with 6/1 x 3.00 mm ACSR conductor werghing 171 Kg/Km on RCC poles of 9m length. The span between adjacent poles is 50m. Make a list of material required and draw a rough sketch of route. | …..2/- |
|  | -2- |  |
| b) | Explain ABC system of rural electrification & state its advantages and disadvantages. |  |
| c) | Explain various types of faults in the system and their effect on distribution system. |  |
| Q.No.5. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Explain PVC casing and capping and state its advantages and disadvantages. |  |
| b) | A class room of size 4m x 4m x 3.5m requires 4 tube lights (40W) and 2 fans (60W). Draw a simple layout showing location of fans, tubes and switchboard. For PVC casing capping estimate total quantity of material required. |  |
| c) | With a neat diagram explain pipe earthing. |  |
| Q.No.6. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | State functions of MCB and ELCB. With a neat diagram explain working of MCB. |  |
| b) | State and explain different method of reducing earth resistance. |  |
| c) | A 3 phase induction motor 400 V, is to be installed for wood cutting in a workshop of size 20mx10m. |  |
|  | 1.Draw a single line and wiring diagram |  |
|  | 2.Find the rating of main switch and motor switch |  |
|  | 3.Find the size of wire for the motor. |  |