## BOARD OF TECHNICAL EDUCATION

PORVORIM-GOA

April,2013 Examinations

Programme: Common

Subject: Elements of Civil Engineering(3003)

# Time Duration: 3 Hrs.                                           Max. Marks: 75

Instructions: 1) All Questions are compulsory.

2) Figures to the right indicate full marks.

3) Assume suitable additional data if required.

| Q.No.1. | Answer any five of the following Questions: | 5 x 3 = 15 |
| --- | --- | --- |
| a) | Explain the method pointwise to measure a straight distance which is about 200m, with help of chain & other instruments. |  |
| b) | Mention some obstructions while measuring distances and explain with a sketch any indirect method of distance measurement across an obstruction. |  |
| c) | Explain with a sketch, method of reciprocal ranging. |  |
| d) | Define bearing of a line. What is meant by Fore bearing and Back bearing? What is the necessity to measure both for each line? |  |
| e) | Convert the following whole circle bearings in to Quadrantal bearings: 175 15, 315 15, 200 15, |  |
| f) | What are the requirements of good roofing for a building? Mention any 6 points. |  |
| g) | Give any formula to decide depth of foundation for a building and a suitable relation between depth and width for a wall footing. |  |
| Q.No.2. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Define: Level line, Horizontal line, B.M., B.S., F.S., I.S., H.I. & R.L. |  |
| b) | Explain point wise temporary adjustment of a theodolite including setting up of tripod. |  |
| c) | Prepare a standard levelling sheet and calculate the levels of all the points using the following readings taken by a dumpy level: 3.52, 3.10, 2.755, 3.005, 3.485, 2.900, 2.225, 1.890, 2.300, 2.155. The instrument is shifted after 4th reading and 7th reading. Take R.L. of first point as 100.00. |  |
| Q.No.3. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Explain stepwise how to prolong a straight line using a theodolite. |  |
| b) | Explain method of measurement of vertical angle by a theodolite. |  |
| c) | Show the classification of foundation and mention any four types of foundations. Mention suitability of each type. |  |
| Q.No.4. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Draw a sketch (plan and elevation) of a grillage foundation. |  |
| b) | Mention any 5 types of flooring and suitability of each type. What is the suitable type of flooring for Industries? |  |
| c) | Draw a neat sketch of King Post or Queen Post roof truss and other roofing members including tiles. Label all the parts. |  |

………2/-

-2-

| Q.No.5. | Answer any two of the following Questions: | 2 x 6 = 12 |
| --- | --- | --- |
| a) | What are the ingredients of concrete? Show some proportions and their suitability where used. |  |
| b) | Explain mixing, placing, compacting, curing, etc to get good concrete structure. |  |
| c) | Explain methods of thermal insulation for an airconditioned building. |  |
| Q.No.6. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | What is the necessity of providing sound insulation to a building? Explain some techniques of sound insulation. |  |
| b) | Draw Civil Engineering symbols for - W.C. Indian - W.C. European - Kitchen Sink - Bath tub - Wash Basin. |  |
| c) | Draw a neat section of wall from foundation to roof for a two storeyed building indicating Plinth level, Sill level, Chajja level, Ceiling level, Ridge level, Eaves level, etc. |  |

***********