## BOARD OF TECHNICAL EDUCATION

PORVORIM-GOA

April, 2013 Examinations

Programme: Electrical Engg./E&EE

Subject: A.C. Machines(4113)

# Time Duration: 3 Hrs.                                               Max. Marks:  75

Instructions: 1) All Questions are compulsory.

2) Figures to the right indicate full marks.

3) Assume suitable additional data if required.

| Q.No.1. | Answer any five of the following: | 5 x 3 = 15 |
| --- | --- | --- |
| a) | What do you mean by ‘Rotating Magnetic Field’? How it is produced in a three phase induction motor? |  |
| b) | What is soft starter? State its advantages. |  |
| c) | Briefly explain working of Reluctance start Induction Motor. |  |
| d) | State advantages of using rotating field and stationary armature in an Alternator. |  |
| e) | State need for parallel operation of Alternators. What are the conditions that must be satisfied before connecting two alternators in parallel? |  |
| f) | What is a synchronous condensor? State its advantages. |  |
| g) | What is an Universal motor? State its applications. |  |
| Q.No.2. | Answer any two of the following: | 2 x 6 = 12 |
| a) | Draw and explain phasor diagram of Induction Motor operating at full load conditions. |  |
| b) | Using the torque equation draw the nature of torque-slip characteristics of three phase Induction Motor. Explain how variation in motor circuit resistance will affect the torque-slip characteristics. |  |
| c) | Explain the principle of working of three phase Induction Motor. Can it run at synchronous speed? |  |
| Q.No.3. | Answer any two of the following: | 2 x 6 = 12 |
| a) | Show how three phase Induction motor can be represented by an equivalent circuit? State importance of equivalent circuit. |  |
| b) | A 400 Volt, 6 pole, 3 –phase, 50Hz Induction motor has following parameters R2=0.545; X2=0.74 R1=0.545; X1=1.48; X0=31.25, R0=263.2. Find the current drawn by stator per phase from supply when the motor is running with a slip of 4. |  |
| c) | The power input to a 400V, 50Hz, 6-pole, 3-phase Induction motor running at 970rpm is 50KW. The stator losses are 1KW & windage and fraction losses are 2 KW. Calculate - Slip - Rotor Copper losses - Efficiency of motor. |  |
| Q.No.4. | Answer any two of the following: | 2 x 6 = 12 |
| a) | With neat diagram explain working of DOL starter. |  |
| b) | Explain construction and working of shaded pole motor. |  |
| c) | Describe the main parts of alternator with their functions. |  |

………2/-

-2-

| Q.No.5. | Answer any two of the following: | 2 x 6 = 12 |
| --- | --- | --- |
| a) | Explain the phenomenon of armature reaction in an alternator with the help of suitable diagrams. |  |
| b) | State the causes of voltage drop in an alternator when loaded. Describe any one method of determining voltage regulation of an alternator. |  |
| c) | A three phase, star connected alternator is rated at 1600KVA & 13500Volt. The per phase effective armature resistance and synchronous reactance are 1.5 & 30 respectively. Calculate the percentage regulation for a load of 1280KW at a power factor of 0.8 lagging. |  |
| Q.No.6. | Answer any two of the following: | 2 x 6 = 12 |
| a) | Explain with the help of phasor diagram effect of change of excitation of any one alternator when it is operating in parallel with another alternator. |  |
| b) | Explain why synchronous motor does not have starting torque. Describe any two methods of starting a synchronous motor. |  |
| c) | Explain working of Hysterisis motor. State its applications. |  |

***********