## BOARD OF TECHNICAL EDUCATION

PORVORIM-GOA

April, 2013 Examinations

Programme: Civil Engg./CT/M

Subject: Hydraulics(4025)

# Time Duration: 3 Hrs.                                               Max. Marks:  75

Instructions: 1) All Questions are compulsory.

2) Figures to the right indicate full marks.

3) Assume suitable additional data if required.

| Q.No.1. | Answer any five of the following Questions: | 5 x 3 = 15 |
| --- | --- | --- |
| a) | Calculate the specific weight, mass density and specific gravity of a liquid having a volume of 8m3 and weight of 48 KN. |  |
| b) | Define ‘Kinematic viscosity’ and state its units. |  |
| c) | Find the depth below water surface in sea, where the pressure intensity is 100 KN/m2. Given specific gravity of seawater is 1.025 times that of fresh water. |  |
| d) | Distinguish between steady flow and unsteady flow. |  |
| e) | Distinguish clearly between an orifice and a mouthpiece. |  |
| f) | Give the advantages of triangular notch or weir over a rectangular notch or weir. |  |
| g) | State the laws of fluid friction. |  |
| h) | Define what is critical flow in case of flow in open channels. |  |
| Q.No.2. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | With the help of a neat sketch explain the construction and working of a Bourdon’s Tube Pressure Gauge. |  |
| b) | The right limb of a simple U-tube manometer containing mercury is open to the atmosphere while the left limb is connected to a pipe in which a liquid of specific gravity 0.8 is flowing. The centre of the pipe is 12 cms above the level of mercury in the left limb. Find the pressure of the liquid in the pipe if the difference of mercury level in the two limbs is 20 cms. |  |
| c) | A rectangular plane surface is 2 m wide and 3 m deep. It lies in vertical plane in oil of specific gravity 0.9. Determine the total pressure and position of centre of pressure on the plane surface when its upper edge is horizontal and coincides with oil surface. |  |
| Q.No.3. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | With the help of a neat sketch explain the working of a Hydraulic Ram. |  |
| b) | The diameters of a pipe at section 1-1 and 2-2 are 300 mm and 600mm respectively. If the velocity of water flowing through the pipe at section 1-1 is 4 m/s, find - The Discharge through the pipe - Velocity of water at section 2-2. |  |
| c) | A 30 cms x 15 cms venturimeter is inserted in a vertical pipe carrying water. The flow of water is in upward direction. The difference of levels between the throat and inlet section is 50 cms. The mercury differential manometer gives a reading of 30 cms of mercury. Find the discharge of water. Neglect losses. |  |
|  |  | ……2/- |
|  | -2- |  |
| Q.No.4. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | A tank has two identical orifices in one of its vertical sides. The upper orifice is 1.5m below the water surface and the lower one is 3m below the water surface. Find the point, at which the two jets will intersect, if the co-efficient of velocity is 0.92 for both the orifices. |  |
| b) | Water is to be supplied to the inhabitants of a college campus through a supply main. The following data is given: Distance of reservoir from the campus = 3000m Number of inhabitants = 4000 Consumption of water per day of each inhabitant =180litres Loss of head due to friction = 18 m Co-efficient of friction for the pipe, f = 0.07 If half the daily supply is pumped in 8 hours, determine the size of the supply main. |  |
| c) | Find the bed slope of trapezoidal channel of bed width 5 m, depth of water 3m and side slope of 2 horizontal to 4 vertical, when the discharge through the channel is 20m3/s. Take Manning’s N =0.03 in Manning’s formula C = 1/N m1/6. |  |
| Q.No.5. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Find the discharge over a Cippoletti weir of length 2m when the head over the weir is 1.2m. Take cd = 0.62. |  |
| b) | Find the discharge from a 200 mm diameter external mouthpiece, fitted to a side of a large vessel if the head over the mouthpiece is 5 metres. |  |
| c) | A piping system consists of three pipes arranged in series. The length of the pipes are 1200m, 750m, 600m and the diameters are 750mm, 600mm and 450mm respectively. - Transform the system to an equivalent 450mm diameter pipe. Find the length of the equivalent pipe. - Determine the diameter of equivalent pipe given its length to be 2550m. |  |
| Q.No.6. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | A rectangular channel of width 6m is having bed slope of 1 in 2000. Find the maximum discharge through the channel. Take value of c = 50. |  |
| b) | Distinguish clearly between centrifugal pump and reciprocating pump from the point of view of selection and principal of working. |  |
| c) | Write short notes on any two of the following: - Classification of Notches and weirs - Hydraulic Co-efficient of orifices - Conditions for most economical rectangular channel & trapezoidal channel. |  |

***********