## BOARD OF TECHNICAL EDUCATION

PORVORIM-GOA

April, 2013 Examinations

Programme: Fabrication Technology & Erection Engg.

Subject: Fabrication Process-I(4233)

# Time Duration: 3 Hrs.                                               Max. Marks:  75

Instructions: 1) All Questions are compulsory.

2) Figures to the right indicate full marks.

3) Assume suitable additional data if required.

| Q.No.1. | Answer any five of the following Questions: | 5 x 3 = 15 |
| --- | --- | --- |
| a) | Large storage tanks are fabricated by welding MS plates. Suggest a suitable way to check vertical alignment of these tanks. |  |
| b) | What is “fundamental deviation” as used in shafts & holes? How is it related to tolerance? |  |
| c) | What is TPI of Hacksaw blade? In metric system how do we specify the same? |  |
| d) | Sketch 3 different types of templates and depict their uses. |  |
| e) | Can items made of cast iron be flame cut? Give reasons. |  |
| f) | Compare line standards with end standards. |  |
| g) | Can spirit level be used to measure slope or angle? How? |  |
| Q.No.2. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Explain dial indicator method of testing squareness. |  |
| b) | What are the sources of errors in measurement? |  |
| c) | Sketch and explain the following types of limit gauges used in industries: i) Snap gauge ii) Plain plug gauge iii) Plain ring gauge. |  |
| Q.No.3. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Write a note on protection and storage of templates and tools. |  |
| b) | With a neat sketch explain the construction and use of beam trammels. |  |
| c) | What is the procedure for marking out a bracket from a datum surface? |  |
| Q.No.4. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Write six important safety precautions to be taken in band machining. |  |
| b) | Describe a reciprocating power hacksaw with the help of an illustration. |  |
| c) | For sheet metal work, what is the basic method of laying out templates? |  |
| Q.No.5. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | What is “backfire” and “flashback” in oxyfuel welding and cutting? How can these be prevented and what are the immediate actions to be taken? |  |
| b) | What is the role of each of the following in oxy-acetylene cutting set: - Two stage oxygen regulator - Tip cleaners - Pressure gauges mounted on cylinders. |  |
| c) | Write notes on: - Procedure for lighting the cutting torch - Procedure for extinguishing the flame. | …2/- |

-2-

| Q.No.6. | Write short notes on any four: | 4 x 3 = 12 |
| --- | --- | --- |
| a) | Circular metal saw |  |
| b) | Making pipeline angle-mitre bends using abrasive cutting wheel |  |
| c) | Trimming rectangular plates by marking critical points & edges to a perfect rectangle |  |
| d) | Hole basis & shaft basis designations of limits of sizes |  |
| e) | Wave length standards |  |
| f) | Geometrical features of a cutter nozzle (Illustrated with a sketch) |  |

***********