## BOARD OF TECHNICAL EDUCATION

PORVORIM-GOA

April, 2013 Examinations

Programme: Electronics & Instrumentation Engg.

Subject: Industrial Instrumentation(4218)

# Time Duration: 3 Hrs.                                           Max. Marks: 75

Instructions: 1) All Questions are compulsory.

2) Figures to the right indicate full marks.

3) Assume suitable additional data if required.

| Q.No.1. | Answer any five of the following: | 5 x 3 = 15 |
| --- | --- | --- |
| a) | State the principle of operation and application of Ultrasonic flow meter. |  |
| b) | Differentiate between three wire and four wire R.T.D’s. What is meant by RTD 500? |  |
| c) | What are the different types of pressure manometers? Explain any one of them. |  |
| d) | Explain the concept of ‘Vena Contracta’ in case of an orifice plate. Also state typical application of segmented orifice plate. |  |
| e) | State the cut-off limit for turbulent and laminar flow. Also explain in brief the working of ‘Bypass Rotameter’. |  |
| f) | Sketch and explain ‘Soap film’ burrete method. |  |
| g) | What is the principle on which Pitot tube works? State one application of this transducer. |  |
| Q.No.2. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Explain the concept of mass flow meter. Justify its need in flow measurements. Also explain mass flow meter with single tube. |  |
| b) | What is meant by cold junction with respect to thermocouple? Briefly explain the three laws of thermoelectricity as applied to thermocouple. |  |
| c) | List out the various types of flow meter. With the help of a diagram explain the working of magnetic flow meter. Discuss the performance. |  |
| Q.No.3. | Answer the following: | 3 x 4 = 12 |
| a) | Sketch and explain the principle and working of an Air purging system. |  |
| b) | Sketch and explain the working of a vapour-pressure thermometer. Also, State the temperature ranges of any two liquids used in such thermometers. |  |
| c) | Discuss in brief any two practical problems that arise during level measurements. Suggest suitable remedy in each case. |  |
| Q.No.4. | Answer the following: |  |
| a) | Differentiate between “Pirani gauge” and “thermocouple vacuum gauge”. Also, discuss McLeod gauge as a standard for pressure measurement. | (5) |
| b) | Discuss the need for electro-pneumatic converters. Sketch and explain working of P to I Converter. | (5) |
| c) | i)Define the terms ‘Lost motion’. ii)Discuss the types of liquids used in manometers. | (2) |

………2/-

-2-

| Q.No.5. | Answer the following Questions: |  |
| --- | --- | --- |
| a) | Discuss any four direct methods of liquid level measurement. | (4) |
| b) | List out the various displacer level detectors. Sketch and explain the working of the Torque tube type displacer, stating its principle. | (5) |
| c) | Discuss the servicing techniques of different types of level measuring instruments. | (3) |
| Q.No.6. | Write short notes on any three: | 3 x 4 = 12 |
| a) | Gaseous Fowmeter calibrations |  |
| b) | Practical applications o pressure control systems |  |
| c) | Voltage to current converter |  |
| d) | Closed loop flow control system |  |
| e) | Filled system thermometer |  |
| f) | Practical applications of temperature control system |  |

***********

õ