## BOARD OF TECHNICAL EDUCATION

PORVORIM-GOA

April, 2013 Examinations

Programme: Mechanical Engg.

Subject: Fluid Machinery(4040)

# Time Duration: 3 Hrs.                                               Max. Marks:  75

Instructions: 1) All Questions are compulsory.

2) Figures to the right indicate full marks.

3) Assume suitable additional data if required.

| Q.No.1. | Answer any five of the following Questions: | 5 x 3 = 15 |
| --- | --- | --- |
| a) | How will you classify the turbines? |  |
| b) | Differentiate between the volute casing and vortex casing for the centrifugal pump. |  |
| c) | How will you classify reciprocating pumps? |  |
| d) | Draw the symbols of the following pneumatic elements: i)Single acting cylinder ii)Safety valve. |  |
| e) | Give the importance of pressure control valves in Hydraulic control systems. |  |
| f) | Derive an expression for the force exerted by the jet on a stationary vertical plate in the direction of the jet. |  |
| g) | Define the terms: suction head, delivery head and manometric head in case of centrifugal pump. |  |
| Q.No.2. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | A pelton wheel is having a mean bucket diameter of 1m and is running at 1000 r.p.m. The net head on the pelton wheel is 700m. If the side clearance angle is 15 and discharge through nozzle is 0.1 m3/s. - Power available at the nozzle - Hydraulic efficiency of the turbine. |  |
| b) | A Kaplan turbine runner is to be designed to develop 7357.5 KW shaft power. The net available head is 5.50m. Assume that the speed ratio is 2.09 and flow ratio is 0.68, and the overall efficiency of 60%. The diameter of the boss is 1/3 rd of the diameter of the runner. Find the diameter of the runner, its speed and its specific speed. |  |
| c) | Define ‘The specific speed of a turbine’. Give the range of specific speed values of the Kaplan, Francis turbines and Pelton wheels. What factors decide whether Kaplan, Francis, or a Pelton type turbine would be used in hydroelectric project? |  |
| Q.No.3. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | A Francis turbine with an overall efficiency of 75% is required to produce 148.25 KW power. It is working under a head of 7.62m. The peripheral velocity = 0.262gHand the radial velocity of flow at inlet is 0.962gH. The wheel runs at 150 r.p.m and the hydraulic losses in the turbine are 22% of the available energy. Assuming radial discharge, determine: - The guide blade angle - The wheel vane angle at inlet - Diameter of the wheel at inlet - Width of the wheel at inlet. |  |
| b) | What do you understand by the characteristic curves of a turbine? Name the important types of characteristic curves and explain any one type in detail. | ……2/- |
|  | -2- |  |
| c) | Define cavitation. What are the effects of cavitation? Give the necessary precautions against cavitation. |  |
| Q.No.4. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | The internal and external diameters of the impeller of a centrifugal pump are 200mm and 400mm respectively. The pump is running at 1200 r.p.m. The vane angles of the impeller at inlet and outlet are 20 and 30 respectively. The water enters the impeller radially and velocity of flow is constant. Determine the work done by the impeller per unit weight of water. |  |
| b) | Why are centrifugal pumps used sometimes in series and some times in parallel? Explain in detail with sketches. |  |
| c) | Derive an expression for the minimum speed for starting a centrifugal pump. |  |
| Q.No.5. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | A double acting reciprocating pump, running at 40 r.p.m. is discharging 1m3 of water per minute. The pump has a stroke of 400mm. The diameter of the piston is 200mm, the delivery and suction heads are 20m and 5m respectively. Find the slip of the pump and power required to drive the pump. |  |
| b) | What is the effect of acceleration in suction and delivery pipes on indicator diagram? Does the area of the indicator diagram change as compared to the area of ideal indicator diagram? |  |
| c) | What is an air vessel? Describe with neat sketches the function of the air vessel for reciprocating pumps. |  |
| Q.No.6. | Answer any four of the following Questions: | 4 x 3 = 12 |
| a) | With the help of a neat sketch explain the working of a rotary vane pump. |  |
| b) | What is a variable delivery pump? What are its applications? |  |
| c) | Draw and explain the regenerative circuits. |  |
| d) | Name and explain the different types of direction control valves. |  |
| e) | Draw the symbols of the following pneumatic elements: - Shuttle valve - Quick exhaust valve - Non-return valve. |  |
| f) | Draw and explain counter balance circuits. |  |
| g) | For pumping LPG, which pump will you use and why? |  |

***********