## BOARD OF TECHNICAL EDUCATION

PORVORIM-GOA

April, 2013 Examinations

Programme: Computer Engg.

Subject: P.C. Hardware(4223)

# Time Duration: 3 Hrs.                                               Max. Marks:  75

Instructions: 1) All Questions are compulsory.

2) Figures to the right indicate full marks.

3) Assume suitable additional data if required.

| Q.No.1. | Answer any five of the following Questions: | 5 x 3 = 15 |
| --- | --- | --- |
| a) | Explain the purpose of expansion slots present on the motherboard. |  |
| b) | List any three features of Pentium Processor which makes it different from 8086 processors. |  |
| c) | Write a note on various buses present on the motherboard. |  |
| d) | Explain the concept of UMA |  |
| e) | Define: i) Resolution ii) Interlacing iii) Refresh Rate. |  |
| f) | Write on different types of a floppy disks & drives. |  |
| Q.No.2. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Explain the working principle of a Trackball mouse. |  |
| b) | What is the function of SMPS in the PC system? Explain the on board power connector of SMPS. |  |
| c) | What is meant by SIMM and DIMM? State the features of different RAM chips associated with SIMM and DIMM. |  |
| Q.No.3. | Answer the following Questions: | 2 x 6 = 12 |
| a) | Explain the working principle of Laser printer. |  |
| b) | Explain the construction and working principle of capacitive key switches with a neat diagram. |  |
| Q.No.4. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Explain the construction of floppy disk drive. |  |
| b) | Explain the working of a video board. |  |
| c) | Explain the read/write operation of a hard disk drive in a PC system. |  |
| Q.No.5. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Explain the construction of hard disk drive. |  |
| b) | Write a note on CD-ROM installation. |  |
| c) | Explain the write operation in CD R/W. |  |
| Q.No.6. | Answer the following Questions: | 2 x 6 = 12 |
| a) | With a neat block diagram explain the functioning of a standard sound card in a PC. |  |
| b) | Explain in brief different components present on the motherboard. |  |

***********