## BOARD OF TECHNICAL EDUCATION

PORVORIM-GOA

April, 2013 Examinations

Programme: Electronics Engg./EC/COMP/EI/MED/E&EE

Subject: Microprocessors-I(8085)[4142]

# Time Duration: 3 Hrs.                                               Max. Marks:  75

Instructions: 1) All Questions are compulsory.

2) Figures to the right indicate full marks.

3) Assume suitable additional data if required.

| Q.No.1. | Answer any five of the following: | 5 x 3 = 15 |
| --- | --- | --- |
| a) | What is the maximum number of memory locations that can be addressed by the P-8085? Why? |  |
| b) | Differentiate between peripheral mapped I/O and memory mapped I/O. |  |
| c) | What are branching instructions? Explain the function of 2 instructions with example. |  |
| d) | Explain (i) stack pointer (ii)Nesting of subroutines. |  |
| e) | Find the content of A (accumulator) after the execution of: (i) MVI A, 4FH (ii) MVI A, 45H ADI 00H SUI 06H DAA DAA |  |
| f) | Design the required connection at terminals (pins) X1 and X2 of P-8085 to operate system at 4 MHz. With such a set up, how much time is required to execute an instruction with 18 T states. |  |
| Q.No.2. | Answer any two of the following: | 2 x 6 = 12 |
| a) | Draw the flag register and explain each flag of P-8085. Illustrate any two instructions that cause content of flag register to change. |  |
| b) | Using a block diagram, explain the memory read operation of P-8085. Specify the control signal and the direction of data flow on the data bus in a memory write operation. |  |
| c) | Draw a logic schematic to generate 4 control signals using the P-8085, IOM, RD and WR signals : (i) MEMR (ii) MEMW (iii) IOR iv) IOW. Explain the function of these derived signals. |  |
| Q.No.3. | Answer any two of the following: | 2 x 6 = 12 |
| a) | Define Instruction. With examples, explain the classification of instructions based on bytes required. |  |
| b) | List the classification of addressing modes used by P-8085 with examples. Explain the operation performed by 3 arithmetic instructions. |  |
| c) | Write a program to (i) Clear accumulator (ii) Add 47H to A (iii) Subtract 92H, display result at Port no.30 (iv) Add 64 H, store result at location 7500H (v) Clear A and terminate program. Show change in flag register with each instruction. |  |
| Q.No.4. | Answer any two of the following: | 2 x 6 = 12 |
| a) | Explain the programming techniques used to instruct the P to repeat tasks. Use flow charts. |  |
| b) | Sixteen bytes of data are stored at memory location 7500H. Write a program, to transfer the block at 7500H to the location 7600H, such that consecutive byte is separated by 00H. | …..2/- |

| c) | Write a program to add a block of 16 data stored in memory location 7600H, also count the number of times carry is generated. Store the count in register C and sum at 7650H. |  |
| --- | --- | --- |
| Q.No.5. | Answer the following: | 2 x 6 = 12 |
| a) | Illustrate: (i) Instructions to implement time delay using loop within loop technique (ii) Use of subroutines in programming. |  |
| b) | Define stack. Explain the instructions used to store and retrieve data from stack. |  |
| Q.No.6. | Answer any two of the following: | 2 x 6 = 12 |
| a) | List and explain the classification of 8085 interrupts. |  |
| b) | Draw and explain implementation of non-vectored interrupts such that RST 6 is inserted on data bus when interrupt occurs. |  |
| c) | Using example of simple display of binary data with L.E.D’s, explain the concept of O/P display. |  |

***********