## BOARD OF TECHNICAL EDUCATION

PORVORIM-GOA

April, 2013 Examinations

Programme: Mechanical Engg./P/A/SB

Subject: Strength of Materials(4032)

# Time Duration: 3 Hrs.                                               Max. Marks:  75

Instructions: 1) All Questions are compulsory.

2) Figures to the right indicate full marks.

3) Assume suitable additional data if required.

| Q.No.1. | Answer any five of the following Questions: | 5 x 3 = 15 |
| --- | --- | --- |
| a) | Define the following: - Elasticity - Plasticity - Toughness. |  |
| b) | Calculate the strain energy stored in a bar 3m long, 40mm wide and 30mm thick when subjected to a tensile load of 75KN. E=200GPa. |  |
| c) | State and explain the Parallel Axis theorem. |  |
| d) | A steel wire of 6mm diameter is bent into a circular shape of 8m radius. Determine the maximum stress induced in the wire. |  |
| e) | A rectangular column 100mm wide and 50mm thick is carrying a vertical load of 100KN at an eccentricity of 20mm in a plane bisecting the thickness. Determine the maximum stress induced in the column. |  |
| f) | A steam boiler 500mm diameter is made up of 8mm thick plates. If the boiler is subjected to an internal pressure of 3 MPa, find the hoop stress induced in the plate. |  |
| g) | State the assumptions in Euler’s theory. |  |
| h) | A simply supported beam of 5m span is subjected to a central load of 15KN. Find the maximum deflection of the beam, given I = 12 x 106 mm4 & E = 200GPa. |  |
| Q.No.2. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | An aluminum tube 50mm OD and 25mm ID is tightly fitted on to a solid steel rod of 25mm diameter. The composite bar is loaded in compression by an axial load ‘P’. Find the stress in aluminum when the stress in steel is 75N/mm2. E steel = 200KN/mm2 & E aluminum = 70 KN/mm2. Also find the total load. |  |
| b) | A straight bar is 400mm long and 15mm diameter for the first 200mm length and 10mm in diameter for the remaining length. If the bar is subjected to an axial pull of 20KN, find the total extension in the bar. E=2 x 105N/mm2. |  |
| c) | A steel bar 3m long, 50mm wide and 15mm deep is subjected to a pull of 150KN in the direction of its length. Find the change in length, width and thickness of the bar. E = 200GPa and =0.3. |  |
| Q.No.3. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | The principal stresses at a point in a bar are 220N/mm2 (tensile) and 150N/mm2 (compressive). Determine the resultant stress in magnitude and direction on a plane inclined at 60 to the axis of major principal stress. Also determine the maximum shear stress in the material at that point. |  |

………2/-

-2-

| b) | A rectangular steel block is 250mm long, 25mm wide and 15mm thick. A load of 3KN is suddenly applied along its length. Calculate the strain energy and modulus of Resilience given E= 200GN/m2. |  |
| --- | --- | --- |
| c) | A cantilever beam 2m long carries point loads of 4KN, 5KN and 6KN at distances 0.5m, 1m and 1.5m respectively from the free end. Calculate and draw the Shear Force and Bending Moment diagram for the beam. |  |
| Q.No.4. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | A simply supported beam 8m long carries point loads of 600N & 800N at a distance of 2m and 5m from the left end support and a uniformly distributed load of 200N/m is between the point loads. Calculate and draw the Shear Force and Bending Moment diagram for the beam. |  |
| b) | Find the Moment of Inertia about the centroidal X and Y axis of a ‘T’ section having 100mm x 25mm flange and 120mm x 25mm web. |  |
| c) | A column 80mm x 80mm has a circular cavity of 40mm diameter. Find the maximum value of load that can be applied at an eccentricity of 10mm with respect to the X-X axis in order that the maximum compressive stress may reach 85N/mm2. |  |
| Q.No.5. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | A 50mm diameter shaft is subjected to a torque of 5KN-m. Find - Maximum shear stress in the shaft - Angle of twist per metre length of the shaft - Shear stress at a distance of 15mm from the axis of the shaft. G = 8 x 104 N/mm2. |  |
| b) | A hollow shaft is required to transmit 350KW at 75rpm. If the shear stress is not to exceed 70N/mm2 and the ID is 0.7 times OD, find the OD and ID of the shaft if the maximum torque is likely to exceed the mean torque by 45%. |  |
| c) | A beam of rectangular section supports a load of 25KN/m uniformly distributed over a span of 4m. If the depth of the beam is twice the width and the maximum bending stress is not to exceed 75N/mm2, find the dimensions of the beam. |  |
| Q.No.6. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | A thin cylindrical shell 4m long has 1.5 m ID and 12mm metal thickness. Calculate the hoop and longitudinal stress induced and also the change in dimensions of the shell if it is subjected to an internal pressure of 2 N/mm2. E = 200KN/mm2 and 1/m = 0.3. |  |
| b) | Find the shortest length ‘l’ for a pin ended steel column having a cross section of 50mm x 90mm square for which the Eulers formula applies. E= 2 x 105N/mm2 and critical limit for stress = 300N/mm2. |  |
| c) | A cantilever beam of length 5m is carrying a uniformly distributed load of ‘W’ KN/m. Assuming rectangular cross section with a depth ‘d’ which is equal to twice the width ‘b’, determine the dimensions of the beam so that the vertical deflection at the free end does not exceed 9mm. Maximum bending stress = 120MPa and G = 200GPa. |  |

***********