## BOARD OF TECHNICAL EDUCATION

PORVORIM-GOA

April, 2013 Examinations

Programme: Shipbuilding Engg.

Subject: Shipdrawing & Calculations(4346)

# Time Duration: 3 Hrs.                                               Max. Marks:  75

Instructions: 1) Q.No.1 is compulsory. Answer any five from the remaining

Questions.

2) Figures to the right indicate full marks.

3) Assume suitable additional data if required.

| Q.1. | Write short note on any three: | 3 x 5 = 15 |
| --- | --- | --- |
| a) | Large angle stability |  |
| b) | Floodable length |  |
| c) | Classification society rules |  |
| d) | Metacentric height |  |
| Q.2. | A ‘T’ section 300x6mm web and 100x8mm flange is welded to the bulkhead of a vessel having thickness 10mm. If the spacing between the stiffeners is 600mm, determine section modulus of the stiffener along with the bulkhead plating (attached plating). | (12) |
| Q.3. | A box barge 60m long and 10m wide floats at a level keel draught of 3m. Its centre of gravity is 2.5m above the keel. Determine the end draughts if an empty fore end compartment 9m long is laid open to the sea. | (12) |
| Q.4. | A ship of 15000 tonne displacement has righting levers of 0, 0.38, 1.0, 1.41 and 1.2m at angles of heel of 0, 15, 30, 45 and 60 respectively and an assumed KG of 7.0m. The vessel is loaded to this displacement but the KG is found to be 6.80 m. - Draw the amended stability curve - Estimate the dynamic stability at 60. | (12) |
| Q.5. | Answer the following: |  |
| a) | Draw a typical view of transverse plain bulkhead | (06) |
| b) | Explain in brief the static and dynamic loads acting on a vessel. | (06) |
| Q.6. | What is meant by cross curves of stability? Explain with the help of neat sketches. Explain how the cross curves of stability is obtained given the lines plan of a vessel. | (12) |
| Q.7. | Answer the following: |  |
| a) | What is meant by floodable length curve? Explain the use of the floodable length curve. | (06) |
| b) | Explain a stiff and tender ship. | (06) |

***********