## BOARD OF TECHNICAL EDUCATION

PORVORIM-GOA

April, 2013 Examinations

Programme: Fabrication Technology & Erection Engg.

Subject: Fabrication Drawing(4234)

# Time Duration: 4 Hrs.                                               Max. Marks:  75

Instructions: 1) All Questions are compulsory.

2) Figures to the right indicate full marks.

3) Assume suitable additional data if required.

4) Use of Structural Engineers handbook SP:6(1) & IS 800 is permissible.

5) All dimensions in the figures are in ‘mm’ unless specified.

| Q.No.1. | Answer any five of the following Questions: | 5 x 3 = 15 |
| --- | --- | --- |
| a) | Calculate the weight of the following: - Steel plate of size 172mm x 720mm x 12mm thick - Rolled steel section ISA 12595 x 8.0mm x 1235mm long. |  |
| b) | Sketch a double riveted (Zig-Zag) lap joint and label the following: i) Edge distance ii) Axial pitch iii) Radial pitch iv) Diagonal pitch v) lap vi) Gross diameter of Rivet. |  |
| c) | Give the symbols of the following welds: i) Single Bevel butt ii) Single J-butt iii) Double V-Butt. |  |
| d) | Give the values of “Ixx”, “tw” and “b” of the following rolled steel sections: i) ISBA 250 @ 39.2 Kg/m ii) ISMC 200 @ 22.1 Kg/m. |  |
| e) | Sketch a screw jack pipe support. |  |
| f) | Explain piping flexibility. How is it achieved? |  |
| g) | Sketch the cross-section of any one of the following rolled steel sections, indicate dimensions clearly. - ISMC 175 @ 19.1 Kg/m - ISBA 175 @ 20.0 Kg/m. |  |
| Q.No.2. | Draw the Fabrication drawing of the Gusset plate at “X” & “Y” and determine the exact length of the member X-Y, in the portion of the truss whose design drawing is shown in Fig.1. | (12) |
|  | ### NOTE FOR FIG.1: - All members used are ISA 5050 X 5.0mm - Gusset plate thickness used is 6.0mm - 8.0mm diameter snap headed rivets are used. |  |

………2/-

-2-

| Q.No.3. | Answer the following: |  |
| --- | --- | --- |
| a) | Draw the Front view, Top view and L.H.S.V. of the portion of the pipeline whose Isometric view is given in Fig.2. | (8) |
| b) | Mention at least four points of difference between a Girder and a Beam. | (4) |
|  | #### OR |  |
| b) | Sketch a Bolted Base Connection (Riveted). | (4) |
| Q.No.4. | Answer the following: |  |
| a) | Draw the Top view, Front view and L.H.S.V. of the support structure whose pictorial line diagram is shown in Fig.3. Also draw the Isometric sketch of each identical members of the above structure. | (8) |
|  | ### NOTE FOR FIG.3: |  |
|  | - All members are ISA 4040 X 4.0mm - All joints are welded joints, butt or Fillet. No Gusset plates are used. |  |
| b) | Make a Bill of Material (BOM) of the support structure, shown in Fig.3,Q.4a. | (4) |
|  | #### OR |  |
| b) | Explain the importance of giving the right specifications of steel rolled sections. Specify a hexagonal bar. | (4) |
| Q.No.5. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Sketch a double strap single Riveted Butt joint, between two plates of thickness 8.00mm & 12mm. Indicate the dimensions clearly. |  |
| b) | Sketch a column splicing of bottom column ISHB 400 @ 77.4 Kg/m. and top column ISHB200 @ 37.3 Kg/m. Indicate dimensions clearly. |  |
| c) | Interpret the symbolic weld representations shown in Fig.4. (any two) |  |
| Q.No.6. | Write notes on any two: | 2 x 6 = 12 |
| a) | Importance of pipe supports in pipelines |  |
| b) | Procedure involved in converting a design drawing of a welded structure into its Fabrication Drawing |  |
| c) | Built – up – members |  |
|  |  | ….3/- |